API Reference Guide Xamarin SDK Ver. 1.18

www.bixolon.com

Copyright

© BIXOLON Co., Ltd. All rights reserved.

This user manual and all property of the product are protected under copyright law.

It is strictly prohibited to copy, store, and transmit the whole or any part of the manual and any property of the product without the prior written approval of BIXOLON Co., Ltd.

The information contained herein is designed only for use with this BIXOLON product.

BIXOLON is not responsible for any direct or indirect damages, arising from or related to use of this information.

• The BIXOLON logo is the registered trademark of BIXOLON Co., Ltd.

• All other brand or product names are trademarks of their respective companies or

organizations.

BIXOLON Co., Ltd. maintains ongoing efforts to enhance and upgrade the functions and quality of all our products.

In the following, product specifications and/or user manual content may be changed without prior notice.

Caution

Some semiconductor devices are easily damaged by static electricity. You should turn the printer “OFF”, before you connect or remove the cables on the rear side, in order to guard the printer against the static electricity. If the printer is damaged by the static electricity, you should turn the printer “OFF”.

Table of Contents

Copyright	2

1. Manual Guide	7

2. Supported Devices	8

3. Xamarin SDK Installation	11

3-1 Supported Platforms	11

3-1-1 iOS	11

3-1-2 Android	11

3-1-3 Windows	11

3-2 Supported Communications	12

3-2-1 Bluetooth	12

3-2-2 WiFi / Ethernet	12

3-2-3 USB	12

3-2-4 BLE	12

3-3 Installation Instruction	15

3-3-1 Installation steps from NuGet server	15

3-3-2 Installation steps from a local nupkg file	16

4. IMPosDeviceFactory Reference	19

4-1 Overview	19

4-2 Methods	19

4-2-1 createDevice	19

5. MPosControllerDevices Reference	20

5-1 Overview	20

5-2 Properties	20

5-2-1 dataEvent	20

5-2-2 statusUpdateEvent	20

5-2-3 DeviceID	21

5-2-4 IsOpen	21

5-2-5 CommandMode	21

5-2-6 InterfaceType	21

5-3 Methods	22

5-3-1 selectInterface	22

5-3-2 selectCommandMode	24

5-3-3 openService	25

5-3-4 closeService	26

5-3-5 directIO	27

5-3-6 setTransaction	28

6. MPosControllerPrinter Reference	29

6-1 Overview	29

6-2 Methods	30

6-2-1 printText	30

6-2-2 printText	31

6-2-3 setCharacterset	32

6-2-4 setInternationalCharacterset	34

6-2-5 setPagemode	35

6-2-6 setPagemodePrintArea	36

6-2-7 setPagemodeDirection	37

6-2-8 setPagemodePosition	38

6-2-9 getFirmwareVersion	39

6-2-10 printBitmap	40

6-2-11 printBitmapFile	41

6-2-12 print1DBarcode	42

6-2-13 printQRCode	44

6-2-14 printPDF417	45

6-2-15 checkPrinterStatus	46

6-2-16 checkBattStatus	47

6-2-17 asbEnable	48

6-2-18 printLine	49

6-2-19 printBox	50

6-2-20 printPdfFile	51

6-2-21 printPdfFile	52

6-2-22 cutPaper	53

6-2-23 openDrawer	54

7. MPosLookup Reference	55

7-1 Overview	55

7-2 Methods	55

7-2-1 refreshDevicesList	55

7-2-2 getDevicesList	56

8. MPosControllerLabelPrinter Reference	57

8-1 Overview	57

8-2 Methods	57

8-2-1 setTextEncoding	57

8-2-2 setCharacterset	59

8-2-3 checkPrinterStatus	61

8-2-4 printBuffer	62

8-2-5 printRawData	63

8-2-6 drawTextDeviceFont	64

8-2-7 drawTextVectorFont	66

8-2-8 drawBarcode1D	68

8-2-9 drawBarcodeMaxiCode	70

8-2-10 drawBarcodePDF417	71

8-2-11 drawBarcodeQRCode	73

8-2-12 drawBarcodeDataMatrix	74

8-2-13 drawBarcodeAztec	75

8-2-14 drawBarcodeCode49	76

8-2-15 drawBarcodeCodaBlock	78

8-2-16 drawBarcodeMicroPDF	79

8-2-17 drawBarcodeIMB	80

8-2-18 drawBarcodeMSI	81

8-2-19 drawBarcodePlessey	82

8-2-20 drawBarcodeTLC39	83

8-2-21 drawBarcodeRSS	84

8-2-22 drawBlock	86

8-2-23 drawCircle	87

8-2-24 drawImage	88

8-2-25 drawImageFile	89

8-2-26 setPrintingType	90

8-2-27 setMargin	91

8-2-28 setLength	92

8-2-29 setWidth	93

8-2-30 setSpeed	94

8-2-31 setDensity	95

8-2-32 setOrientation	96

8-2-33 setOffset	97

8-2-34 setCuttingPosition	98

8-2-35 setAutoCutter	99

8-2-36 getModelName	100

8-2-37 getFirmwareVersion	101

8-2-38 getPrinterInformation	102

8-2-39 setupRFID	103

8-2-40 setRFIDPosition	104

8-2-41 setRFIDPassword	105

8-2-42 setEPCDataStructure	106

8-2-43 writeRFID	107

8-2-44 lockRFID	108

8-2-45 drawPDFFile	109

9. MPosControllerBCD Reference	110

9-1 Overview	110

9-2 Method	110

9-2-1 displayString	110

9-2-2 cleanScreen	113

9-2-3 storeImage	114

9-2-4 storeImageFile	115

9-2-5 displayImage	116

9-2-6 clearImage	117

9-2-7 showCursor	118

10. MPosControllerDallasKey Reference	119

10-1 Overview	119

11. MPosControllerHID Reference	120

11-1 Overview	120

12. MPosControllerMSR Reference	121

12-1 Overview	121

13. MPosControllerNFC Reference	122

13-1 Overview	122

14. MPosControllerRFID Reference	123

14-1 Overview	123

15. MPosControllerScale Reference	124

15-1 Overview	124

16. MPosControllerScanner Reference	125

16-1 Overview	125

17. MPosControllerUSBSerial Reference	126

17-1 Overview	126

18. MPosControllerConfig Reference	127

18-1 Overview	127

18-2 Methods	127

18-2-1 searchDevices	127

18-2-2 getBGateSerialNumber	128

18-2-3 getUSBDevice	129

18-2-4 getCustomDevices	130

18-2-5 getSerialConfiguration	131

18-2-6 setSerialConfiguration	132

18-2-7 addCustomDevice	133

18-2-8 deleteCustomDevice	134

18-2-9 reInitCustomDeviceType	135

18-2-10 getSerialNumber	136

19. Appendix	137

19-1 Error Code Table	137

1. Manual Guide

This SDK manual provides descriptions of contents necessary for developing Xamarin.Forms (Portable Class Library, NET Standard) applications. Xamarin library and Demo application were built using Visual Studio 2015 on Windows, and all the contents in this manual are explained based on Visual Studio 2015 on Windows.

2. Supported Devices

The below table summarizes the supported devices (printers) that are available in this Xamarin SDK.

| Receipt Printers | Resolution | Supported Communication | Max Printable Width |
| --- | --- | --- | --- |
| SPP-R200II | 203 dpi | Bluetooth, Wi-Fi, USB | 384 dots |
| SPP-R200III | 203 dpi |  | 384 dots |
| SPP-R210 | 203 dpi |  | 384 dots |
| SPP-R300 | 203 dpi |  | 576 dots |
| SPP-R310 | 203 dpi |  | 576 dots |
| SPP-R400 | 203 dpi |  | 832 dots |
| SPP-R410 | 203 dpi |  | 832 dots |
| SPP-C200 | 203 dpi | Bluetooth, USB | 384 dots |
| SPP-C300 | 203 dpi |  | 576 dots |
| SRP-350IIOBE | 180 dpi | Bluetooth, Ethernet, USB | 512 dots |
| SRP-350plusIII | 180 dpi | Bluetooth, Ethernet, Wi-Fi, USB | 512 dots |
| SRP-352plusIII | 203 dpi |  | 576 dots |
| SRP-350plusV | 180 dpi |  | 512 dots |
| SRP-352plusV | 203 dpi |  | 576 dots |
| SRP-380 | 180 dpi |  | 512 dots |
| SRP-382 | 203 dpi |  | 576 dots |
| SRP-383 | 300 dpi |  | 864 dots |
| SRP-380II | 180 dpi |  | 512 dots |
| SRP-382II | 203 dpi |  | 576 dots |
| SRP-Q200 | 203 dpi |  | 432 dots |
| SRP-Q300 | 180 dpi |  | 512 dots |
| SRP-Q302 | 203 dpi |  | 576 dots |
| SRP-Q300II | 203 dpi |  | 576 dots |
| SRP-F310II | 180 dpi |  | 512 dots |
| SRP-F312II | 203 dpi |  | 576 dots |
| SRP-F313II | 203 dpi |  | 640 dots |
| SRP-S300 | 203 dpi |  | 576 dots |
| SRP-S320 | 203 dpi |  | 576 dots |
| SRP-350III | 180 dpi | Ethernet, USB | 512 dots |
| SRP-352III | 203 dpi |  | 576 dots |
| SRP-350V | 180 dpi |  | 512 dots |
| SRP-352V | 203 dpi |  | 576 dots |
| SRP-330II | 180 dpi |  | 512 dots |
| SRP-330III | 180 dpi |  | 512 dots |
| SRP-332II | 203 dpi |  | 576 dots |
| SRP-332III | 203 dpi |  | 576 dots |
| SRP-S200 | 203 dpi |  | 432 dots |
| SRP-QE300 | 180 dpi |  | 512 dots |
| SRP-QE302 | 203 dpi |  | 576 dots |
| SRP-E300 | 180 dpi |  | 512 dots |
| SRP-E302 | 203 dpi |  | 576 dots |
| SRP-S3000 | 203 dpi | Ethernet, Wi-Fi, USB | 576 dots |
| BK3-21 | 203 dpi | USB | 384 dots |
| BK3-31 | 203 dpi | USB | 576 dots |
| BK3-32 | 203 dpi | USB | 576 dots |
| BK5-3 | 203 dpi | USB | 576 dots |
| BGT-100P | 180 dpi | Bluetooth, Wi-Fi, Ethernet, USB | 512 dots |
| BGT-102P | 203 dpi |  | 576 dots |
| SRP-Q300H, | 180 dpi |  | 512 dots |
| SRP-Q302H | 203 dpi |  | 576 dots |
| SRP-F310IIHi | 180 dpi |  | 512 dots |
| SRP-F312IIHi | 203 dpi |  | 576 dots |
| SRP-S300Hi | 180 dpi |  | 512 dots |
| SRP-S320Hi | 203 dpi |  | 576 dots |

| Label Printers | Resolution | Supported Communication | Max Printable Width |
| --- | --- | --- | --- |
| SLP-DX220 | 203 dpi | Bluetooth, Wi-Fi, Ethernet, USB | 432 dots |
| SLP-DX223 | 300 dpi |  | 672 dots |
| SLP-TX220 | 203 dpi |  | 432 dots |
| SLP-TX223 | 300 dpi |  | 672 dots |
| SLP-DX420 | 203 dpi |  | 864 dots |
| SLP-DX423 | 300 dpi |  | 1248 dots |
| SLP-TX420 | 203 dpi |  | 864 dots |
| SLP-TX423 | 300 dpi |  | 1248 dots |
| SLP-TX400(RFID) | 203 dpi |  | 864 dots |
| SLP-TX403(RFID) | 300 dpi |  | 1248 dots |
| SRP-770III | 203 dpi |  | 832 dots |
| SLP-DL410 | 203 dpi |  | 864 dots |
| SLP-DL413 | 300 dpi |  | 1248 dots |
| XT5-40(RFID) | 203 dpi |  | 832 dots |
| XT5-43(RFID) | 300 dpi |  | 1248 dots |
| XT5-46(RFID) | 600 dpi |  | 2496 dots |
| XD5-40d | 203 dpi |  | 864 dots |
| XD5-43d | 300 dpi |  | 1248 dots |
| XD5-40t(RFID) | 203 dpi |  | 864 dots |
| XD5-43t(RFID) | 300 dpi |  | 1248 dots |
| XL5-40 | 203 dpi |  | 864 dots |
| XL5-43 | 300 dpi |  | 1248 dots |
| XT3-40 | 203 dpi |  | 864 dots |
| XT3-43 | 300 dpi |  | 1248 dots |
| SRP-E770III | 203 dpi | Ethernet, USB | 832 dots |
| SPP-L3000 | 203 dpi | Bluetooth, Wi-Fi, USB | 576 dots |
| SPP-L310 | 203 dpi |  | 576 dots |
| SPP-L410 | 203 dpi |  | 832 dots |
| XM7-20 | 203 dpi | Bluetooth, Wi-Fi, * Ethernet (Cradle Required), USB | 384 dots |
| XM7-40(RFID) | 203 dpi |  | 832 dots |
| XM7-30 | 203 dpi | Bluetooth, Wi-Fi, USB | 576 dots |
| SRP-S3000_LABEL | 203 dpi | Wi-Fi, Ethernet, USB | 576 dots |
| XQ-840II | 203 dpi | USB | 864 dots |
| XQ-843II | 300 dpi | USB | 1248 dots |

3. Xamarin SDK Installation

3-1 Supported Platforms

3-1-1 iOS

iOS 12.0 or later

In your Xamarin project for iOS, set Deployment Target version to 12.0.

3-1-2 Android

Android 6.0(API Level 23: Marshmallow) or later

In your Xamarin project for android, set minimum android version to API Level 23.

You should leave the target Android version set to Use Compile using SDK version (as shown) so that its value automatically matches the target framework setting. In general, the Target Android Version should be bounded by the Minimum Android Version and the Target Framework.

That is: Minimum Android Version < Target Android Version < Target Framework.

3-1-3 Windows

Windows 10 version Build 14393 or later.

In your Xamarin project for Universal Windows, set minimum version to Winddows 10 (10.0 Build 14393) as below.

3-2 Supported Communications

3-2-1 Bluetooth

HOST must be paired with Bluetooth device for Bluetooth communication in advance. Make sure to include necessary permissions (or capabilities) to each platform before using APIs in this manual. Please refer to the table following for permission settings.

3-2-2 WiFi / Ethernet

HOST and device (printer) can only communicate over TCP / IP on the same wired / wireless network. Make sure to include necessary permissions (or capabilities) to each platform before using APIs in this manual. Please refer to “3-3” in this manual for permission settings.

3-2-3 USB

USB communication is possible while the HOST and device (printer) are connected to each other with a USB cable

3-2-4 BLE

For BLE communication, it is possible to communicate with a Bluetooth device (printer) without pairing with the host, and the Bluetooth device (printer) must support BLE mode. Before using APIs in this manual. Please refer to the table following for permission settings.

| Platform | Required permission / capabilities |
| --- | --- |
| iOS | 1. Update the Info.plist file by opening it in XML editor or text editor. Add the following to the file. This is to allow your app to access the Bluetooth, BLE and Network ports. <key>UISupportedExternalAccessoryProtocols</key> <array><string>com.bixolon.protocol</string></array> <key>NSBluetoothAlwaysUsageDescription</key> <string>Communication with the printer for printing.</string> <key>NSBluetoothPeripheralUsageDescription</key> <string>Communication with the printer for printing.</string>. |
|  | 2. Put a checkmark on “External Accessory communication” checkbox as a below. Multicast Networking Entitlement Request If the iOS project deployment target version is 12 or higher or the iPhone/iPad OS version is 14 or higher, you must need to request multicast permission from Apple to discover printer devices. See the link below. 1) https://developer.apple.com/news/?id=0oi77447 2) https://developer.apple.com/contact/request/networking-multicast [Note] - IP address used for discovering devices: 255.255.255.255 - Port numbers used for discovering devices: 48780, 48781, 9000, 3337a |

| Android | Make sure to include following capabilities in the AndroidManifest.xml file. - BLUETOOTH, BLUETOOTH_ADMIN, BLUETOOTH_PRIVILEGED - ACCESS_WIFI_STATE, CHANGE_WIFI_STATE - ACCESS_FINE_LOCATION or ACCESS_COARSE_LOCATION <manifest xmlns:android="http://schemas.android.com/apk/res/android" android:installLocation="auto"> <uses-sdk android:minSdkVersion="15" /> <uses-permission android:name="android.permission.BLUETOOTH" /> <uses-permission android:name="android.permission.BLUETOOTH_ADMIN" /> <uses-permission android:name="android.permission.BLUETOOTH_PRIVILEGED" /> <uses-permission android:name="android.permission. ACCESS_WIFI_STATE " /> <uses-permission android:name="android.permission. CHANGE_WIFI_STATE " /> <uses-permission android:name="android.permission. ACCESS_FINE_LOCATION" /> </manifest>. |
| --- | --- |
| Windows | Make sure to include following capabilities in the “Package.appxmanifest” file. - Bluetooth, Internet (Client), Private Networks (Client & Server) |

|  | When using Bluetooth, BLE and WiFi, there are a few things to modify your project’s settings and configuration information related to permissions for each platform. Demo application provided helps you modifying the necessary settings of your own Xamarin application. |
| --- | --- |

3-3 Installation Instruction

• Xamarin SDK can be installed as a NuGet package into MS Visual Studio.

When you use NuGet to install a package, it copies the library files to your solution

and automatically updates your project (add references, change config files, etc).

If you remove a package, NuGet reverses whatever changes it maded. If you need

more information about NuGet, visit at https://www.nuget.org/

|  | Before installing NuGet Package for Bixolon Xamarin SDK, library references below must be removed if you added them manually on each platform. |
| --- | --- |

3-3-1 Installation steps from NuGet server

This way isn’t supported yet. Refer to “3-3-2” below in this munual to install Xamarin SDK.

3-3-2 Installation steps from a local nupkg file

1) Open Visual Studio, and open your own Xamarin.Forms solution.

2) From the Solution Explorer, right click the top level solution.

3) Click “Manage NuGet Packages for Solution…”

4) On the top of the right side, click “Setting” button in red.

5) Click “Package Sources”, and then click “+” button.

6) Browse to the directory where “Xam.Plugin.BxlMpXamarinSDK.x.x.x.nupkg” file

exists, and then enter any name you want.

|  | “x.x.x” is the version number for the SDK. |
| --- | --- |

7) Click “Update”

8) Click “OK”.

9) “Xam.Plugin.BxlMpXamarinSDK” package can be found when you select the name

you entered in step 6 in the “Package source” list.

10) Click “Project” checkbox to install the entire package.

11) Click “Install” to start installing all the appropriate references to the libraries for

each platform project.

4. IMPosDeviceFactory Reference

4-1 Overview

• "IMPosDeviceFactory" interface can be used to create an instance implementing

"MPosControllerDevices" interface for communication with a specific device.

All instances for device (printer) control are created using the "createDevice"

method.

4-2 Methods

|  | Use the “current” property in “MPosDeviceFactory” class to call the methods defined in this interface. eg) MPosDeviceFactory.Current.createDevice(10) |
| --- | --- |

4-2-1 createDevice

Gets an instance for communication with a specific device.

[Syntax]

MPosControllerDevices createDevice (MPosDeviceType deviceType);

[Parameters]

MPosDevice deviceType: A value of the device type to create.

| MPosDeviceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_DEVICE_LABEL_PRINTER | 1 | Label Prniter (SLCS) |
| MPOS_DEVICE_PRINTER | 10 | POS Printer (ESC/POS) |

[Returns]

A reference to the instance implementing “MPosControllerDevices” interface specified or null.

5. MPosControllerDevices Reference

5-1 Overview

• “MPosControllerDevices” interface provides common member methods with regards

to the device, and the properties and methods can be used in all the classes

implementing this interface.

5-2 Properties

5-2-1 dataEvent

If data entry occurs in a device, the data in byte array will be sent to the application through this event.

5-2-2 statusUpdateEvent

If the status of a printer device changes, the new status will be sent to the application through the this event. The sent value is an integer type constant value that has been defined in advance.

| MPosPrinterStatus Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_STATUS_NORMAL | 0 | Printing is possible |
| MPOS_STATUS_PRINTER_COVEROPEN | 1 | Cover open |
| MPOS_STATUS_PRINTER_PAPEREMPTY | 2 | No paper |
| MPOS_STATUS_PRINTER_PAPER_NEAREND | 4 | Insufficient paper |
| MPOS_STATUS_PRINTER_ERROR | 8 | Offline (The device is disconnected with HOST for some reasons) |
| MPOS_STATUS_BATT_HIGH | 16 | Battery level is in high |
| MPOS_STATUS_BATT_MID | 32 | Battery level is in middle |
| MPOS_STATUS_BATT_LOW | 64 | Battery level is in low |
| MPOS_STATUS_PRINTER_OFFLINE | 128 | Offline by the printer's power off |

|  | The values related to battery level can be detected on SRP-Q300 and SRP-Q302. |
| --- | --- |

5-2-3 DeviceID

The device Id of the device.

5-2-4 IsOpen

The device in the usable status or not.

5-2-5 CommandMode

Communication way to the device via “B-gate” or directly connection.

5-2-6 InterfaceType

Communication Interface such as Bluetooth, Wireless LAN, Wired Lan, USB or BLE.

5-3 Methods

5-3-1 selectInterface

Configures the interface type and address of the device(printer) to connect.

|  | This method must be used before calling the “openService” method. |
| --- | --- |

[Syntax]

int selectInterface (int interfaceType, string address);

[Parameters]

• int interface type: communication type to connect the device.

| MPosInterfaceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_INTERFACE_WIFI | 1 | WiFi. |
| MPOS_INTERFACE_ETHERNET | 2 | Ethernet. |
| MPOS_INTERFACE_BLUETOOTH | 4 | Bluetooth. |
| MPOS_INTERFACE_BLE | 8 | BLE. |
| MPOS_INTERFACE_USB | 16 | USB |

• string address:

In case of TCP (Wi-Fi or Ethernet) communication, it should be used in the form of "IP address [: port number]". Port number is 9100 by default and can be omitted.

In case of Bluetooth communication, the values described in the table below must be entered depending on the platform.

| Platform | Address value |
| --- | --- |
| iOS | Bluetooth MAC Address |
| Android | Bluetooth MAC Address |
| Windows | Bluetooth MAC Address |

In case of BLE communication, the values described in the table below must be entered depending on the platform.

| Platform | Address value |
| --- | --- |
| iOS | Bluetooth MAC Address |
| Android | Bluetooth MAC Address |
| Windows | Not supported |

In case of USB communication, the values described in the table below must be entered depending on the platform.

| Platform | Address value |
| --- | --- |
| iOS | Not supported |
| Android | Value of USBName |
| Windows | Not supported |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

5-3-2 selectCommandMode

Select whether to communicate directly with the device or via "B-gate".

[Syntax]

int selectCommandMode (int commandMode);

[Parameters]

• int commandMode: communication way with the device

| MPosCommandMode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_MODE_BGATE | 0 | Communicate through B-gate (default) |
| MPOS_MODE_DIRECT | 1 | Direct communication |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

5-3-3 openService

Establishes a communication connection with the device.

[Syntax]

Task<int> openService (uint timeout = 3);

[Parameters]

• uint timeout: Timeout value for connecting a device.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

5-3-4 closeService

Disconnects the communication with the device.

[Syntax]

Task<int> closeService (int closeTimeout = 0);

[Parameters]

• int closeTimeout: Timeout value for disconnecting a device.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

5-3-5 directIO

Sends user-defined data to the device.

[Syntax]

Task<int> directIO(byte[] data);

[Parameters]

• byte[] data: Data to be sent to device.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

5-3-6 setTransaction

Enters or Leaves 'Transaction' mode, and then sends the data in the buffer.

|  | It is recommended to use the Transaction mode when using output devices such as printers, Customer displays, label printers, etc. |
| --- | --- |

[Syntax]

Task<int> setTransaction (int mode);

[Parameters]

• int mode: Configure the transaction mode

| MPosTransactionMode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_PRINTER_TRANSACTION_OUT | 0 | Cancel transaction mode after data transmission |
| MPOS_PRINTER_TRANSACTION_IN | 1 | Enter transaction mode |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6. MPosControllerPrinter Reference

6-1 Overview

• The MPosControllerPrinter interface provides methods for controlling a thermal

printer.

6-2 Methods

6-2-1 printText

Prints characters by specifying various font attributes of the character.

[Syntax]

Task<int> printText (string data, MPosFontAttribute attribute);

[Parameters]

• string data: Character string to be printed.

• MPosFontAttribute: Class Oject for the font attributes.

class MPosFontAttribute { public MPosAlignment Alignment { get; set; } public bool Bold { get; set; } public int CodePage { get; set; } public MPosFontType FontType { get; set; } public MPosFontSizeHeight Height { get; set; } public bool Reverse { get; set; } public MPosFontUnderline Underline { get; set; } public MPosFontSizeWidth Width { get; set; } } Properties • Alignment: specifies characters alignments (Left, Center, Right) • Bold: specifies whether the font is bold. • CodePage: code page used for endcoding a character string. Note: ‘CodePage’ value should be one of ‘MPosCodePage’ or ‘MPosEastAsiaCodePage’ enumeration. • FontType: specifies one of the font types (Font A, Font B, Font C) • Height: specifies the font height. • Width: specifies the font width. • Reverse: specifies whether the font is reversed. • Underline: specifies whether the font is underlined.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-2 printText

Prints characters by specifying various font attributes of the character.

[Syntax]

Task<int> printText(byte[] data, MPosFontAttribute attribute);

[Parameters]

• byte[] data: An array of bytes in a character string encoded to be printed.

• MPosFontAttribute: Class Oject for the font attributes.

class MPosFontAttribute { public MPosAlignment Alignment { get; set; } public bool Bold { get; set; } public int CodePage { get; set; } public MPosFontType FontType { get; set; } public MPosFontSizeHeight Height { get; set; } public bool Reverse { get; set; } public MPosFontUnderline Underline { get; set; } public MPosFontSizeWidth Width { get; set; } } Properties • Alignment: specifies characters alignments (Left, Center, Right) • Bold: specifies whether the font is bold. • CodePage: no used (not vaild in this method). • FontType: specifies one of the font types (Font A, Font B, Font C) • Height: specifies the font height. • Width: specifies the font width. • Reverse: specifies whether the font is reversed. • Underline: specifies whether the font is underlined.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-3 setCharacterset

Sets the code page used for encoding characters.

[Syntax]

Task<int> setCharacterset (int characterset);

[Parameters]

• int characterset: Code page to use when encoding a character string.

| MPosCodePage Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_CODEPAGE_CP437 | 437 | USA, Standard Europe (PC437) |
| MPOS_CODEPAGE_CP850 | 850 | Western European (PC850) |
| MPOS_CODEPAGE_CP860 | 860 | Portuguese (PC860) |
| MPOS_CODEPAGE_CP863 | 863 | Canadian-French (PC863) |
| MPOS_CODEPAGE_CP865 | 865 | Nordic (PC865) |
| MPOS_CODEPAGE_WPC1252 | 1252 | Latin 1 (WPC1252) |
| MPOS_CODEPAGE_CP866 | 866 | Cyrillic #2 (PC866) |
| MPOS_CODEPAGE_CP852 | 852 | Latin 2 (PC852) |
| MPOS_CODEPAGE_CP858 | 858 | Euro (PC858) |
| MPOS_CODEPAGE_CP862 | 862 | Hebrew DOS code (PC862) |
| MPOS_CODEPAGE_THAI11 | 34 | Thai character code 11 (THAI 11) |
| MPOS_CODEPAGE_THAI14 | 31 | Thai character code 14 (THAI 14) |
| MPOS_CODEPAGE_THAI16 | 39 | Thai character code 16 (THAI 16) |
| MPOS_CODEPAGE_THAI18 | 35 | Thai character code 18 (THAI 18) |
| MPOS_CODEPAGE_THAI42 | 23 | Thai character code 42 (THAI 42) |
| MPOS_CODEPAGE_WPC1253 | 1253 | Greek (WPC1253) |
| MPOS_CODEPAGE_WPC1254 | 1254 | Turkce (WPC1254) |
| MPOS_CODEPAGE_WPC1257 | 1257 | Baltic (WPC1257) |
| MPOS_CODEPAGE_WPC1251 | 1251 | Cyrillic (WPC1251) |
| MPOS_CODEPAGE_FARSI | 27 | Persian |
| MPOS_CODEPAGE_FARSI_II | 28 | Persian |
| MPOS_CODEPAGE_CP737 | 737 | Greek (PC 737) |
| MPOS_CODEPAGE_CP775 | 775 | Baltic (PC775) |
| MPOS_CODEPAGE_WPC1255 | 1255 | Hebrew New code (WPC1255) |
| MPOS_CODEPAGE_CP855 | 855 | Cyrillic (PC855) |
| MPOS_CODEPAGE_CP857 | 857 | Turkce (PC857) |
| MPOS_CODEPAGE_CP928 | 928 | Greek (PC928) |
| MPOS_CODEPAGE_WPC1258 | 1258 | Vietnamese (PC1258) |
| MPOS_CODEPAGE_WPC1250 | 1250 | Central European (PC1250) |
| MPOS_CODEPAGE_LATIN9 | 28605 | Latin 9 (Latin 9) |
| MPOS_CODEPAGE_TCVN | 49 | Vietnamese (TCVN3) |
| MPOS_CODEPAGE_TCVN_CAPTIAL | 50 | Vietnamese (TCVN3 Capital) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-4 setInternationalCharacterset

Sets the character set used for encoding international characters.

[Syntax]

Task<int> setInternationalCharacterset (int internationalCharset);

[Parameters]

• int internationalCharset: International character set

| MPosICS Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ICS_USA | 0 | USA Code |
| MPOS_ICS_FRANCE | 1 | FRANCE Code |
| MPOS_ICS_GERMANY | 2 | GERMANY Code |
| MPOS_ICS_UK | 3 | UK Code |
| MPOS_ICS_DENMARK_I | 4 | DENMARK1 Code |
| MPOS_ICS_SWEDEN | 5 | SWEDEN Code |
| MPOS_ICS_ITALY | 6 | ITALY Code |
| MPOS_ICS_SPAIN_I | 7 | SPAIN1 Code |
| MPOS_ICS_JAPAN | 8 | JAPAN Code |
| MPOS_ICS_NORWAY | 9 | NORWAY Code |
| MPOS_ICS_DENMARK_II | 10 | DENMARK 2 Code |
| MPOS_ICS_SPAIN_II | 11 | SPAIN 2 Code |
| MPOS_ICS_LATIN_AMERICA | 12 | LATIN AMERICA Code |
| MPOS_ICS_KOREA | 13 | KOREA Code |
| MPOS_ICS_SLOVENIA_CROATIA | 14 | SLOVENIA Code |
| MPOS_ICS_CHINA | 15 | CHINA Code |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-5 setPagemode

Enters or Leaves 'Page' mode, and then sends the data in the buffer.

|  | Pagemode cannot be used simultaneously with transaction mode. |
| --- | --- |

[Syntax]

Task<int> setPagemode (int mode);

[Parameters]

• int mode: Enter or leave page mode

| MPosPageMode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_PRINTER_PAGEMODE_IN | 1 | Enter Pagemode |
| MPOS_PRINTER_PAGEMODE_OUT | 0 | Leave Pagemode and then start printing |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-6 setPagemodePrintArea

Sets the printing area of pagemode.

[Syntax]

int setPagemodePrintArea (int x, int y, int width, int height);

[Parameters]

• int x: The starting X axis coordinate

• int y: The starting Y axis coordinate

• int width: Printing Width

• int height: Printing Height

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-7 setPagemodeDirection

Sets the printing direction in pagemode.

[Syntax]

int setPagemodeDirection (int direction);

[Parameters]

• int direction: Printing direction.

| MPosPageMode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_PRINTER_PD_0 | 0 | 0 degree turn |
| MPOS_PRINTER_PD_LEFT90 | 1 | 90 degree turn counter-clockwise |
| MPOS_PRINTER_PD_180 | 2 | 180 degree turn |
| MPOS_PRINTER_PD_RIGHT90 | 3 | 90 degree turn clockwise |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-8 setPagemodePosition

Sets the coordinates of the printing target in pagemode.

[Syntax]

int setPagemodePosition (int x, int y);

[Parameters]

• int x: x-coordinate, in dot unit, of the object (text, barcode, image) to be printed.

• int y: y-coordinate, in dot unit, of the object (text, barcode, image) to be printed.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-9 getFirmwareVersion

Gets the printer’s firmware version.

[Syntax]

Task<int> getFirmwareVersion (Action<string> versionDelegate)

[Parameters]

• Action<string> versionDelegate: A delegate with a parameter used for getting a firmware version.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-10 printBitmap

Prints an image using a platform dependent image object.

[Syntax]

Task<int> printBitmap (object platformImage, int width, int alignment, int brightness,

bool isDithering, bool isCompress);

[Parameters]

• object platformImage: A platform dependent image object.

|  | iOS | UIKit.UIImage |
| --- | --- | --- |
|  | Android | Android.Graphics.Bitmap |
|  | Windows | Windows.UI.Xaml.Media.Imaging.WriteableBitmap Windows.UI.Xaml.Media.Imaging.BitmapImage (deprecated) |

• int width: Image width to be printed or original width ot the image.

| MPosImageWidth Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_IMAGE_WIDTH_ASIS | -2 | Print as original image size |

• int alignment: Alignment.

| MPosAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ALIGNMENT_LEFT | 0 | Align to the left |
| MPOS_ALIGNMENT_CENTER | 1 | Align at the center |
| MPOS_ALIGNMENT_RIGHT | 2 | Align to the right |

• int brightness: The brightness level

| Platform | Brightness Range |
| --- | --- |
| Android, UWP | 0 ≤ brightness ≤ 100 |
| iOS | -100 ≤ brightness ≤ 100 |

• bool isDithering: Whether or not to apply dithering

• bool isCompress: Whether or not to compress data

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-11 printBitmapFile

Prints Bitmap file specified by a local path.

[Syntax]

Task<int> printBitmapFile (string path, int width, int alignment, int brightness,

bool isDithering, bool isCompress);

[Parameters]

• string path: The path of the image file.

|  | In Android, the image file should be in the SD card. |
| --- | --- |

• int width: Image width to be printed or original width ot the image.

| MPosImageWidth Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_IMAGE_WIDTH_ASIS | -2 | Print as original image size |

• int alignment: Alignment.

| MPosAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ALIGNMENT_LEFT | 0 | Align to the left |
| MPOS_ALIGNMENT_CENTER | 1 | Align at the center |
| MPOS_ALIGNMENT_RIGHT | 2 | Align to the right |

• int brightness: The brightness level

| Platform | Brightness Range |
| --- | --- |
| Android, UWP | 0 ≤ brightness ≤ 100 |
| iOS | -100 ≤ brightness ≤ 100 |

• bool isDithering: Whether or not to apply dithering

• bool isCompress: Whether or not to compress data

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-12 print1DBarcode

Prints a 1-D barcode.

[Syntax]

Task<int> print1DBarcode (string data, int symbology, int height, int barWidth, int

alignment, int textPosition);

[Parameters]

• string data: data of the barcode to be printed

• int symbology: The type of 1-D barcode.

| MPos1dCodeType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_UPCA | 101 | UPC A barcode |
| MPOS_BARCODE_UPCE | 102 | UPC E barcode |
| MPOS_BARCODE_EAN8 MPOS_BARCODE_JAN8 | 103 | EAN 8/JAN 8 barcode |
| MPOS_BARCODE_EAN13 MPOS_BARCODE_JAN13 | 104 | EAN 13/JAN 13 barcode |
| MPOS_BARCODE_ITF | 106 | ITF barcode |
| MPOS_BARCODE_CODABAR | 107 | CODABAR barcode |
| MPOS_BARCODE_CODE39 | 108 | CODE39 barcode |
| MPOS_BARCODE_CODE93 | 109 | CODE93 barcode |
| MPOS_BARCODE_CODE128 | 110 | CODE128 barcode |

|  | Code128 subset A, B, or C can be printed by entering characters such as "{A", "{B" or "C" at the start of the data. Data in Code128 subset C is only allowed in numbers and must consist of even data. |
| --- | --- |

• int height: The barcode height. (Range: 1 ≤ height ≤ 255)

• int barWidth: The barcode width. (Range: 2 ≤ barWidth ≤ 6)

• int alignment: Alignment value.

| MPosAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ALIGNMENT_LEFT | 0 | Align left |
| MPOS_ALIGNMENT_CENTER | 1 | Align center |
| MPOS_ALIGNMENT_RIGHT | 2 | Align right |

• int textPosition: The printing position of the HRI (Human Readable Interface).

| MPosLabelHRI Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_TEXTNONE | 0 | Does not print |
| MPOS_BARCODE_TEXTABOVE | 1 | Print above barcode |
| MPOS_BARCODE_TEXTBELOW | 2 | Print below barcode |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-13 printQRCode

Prints the QR Code.

[Syntax]

int printQRCode (string data, int model, int alignment, int moduleSize, int eccLevel);

[Parameters]

• string data: The QR Code data

• int model: The model type

| MPos2dCodeType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_QRCODE_MODEL1 | 204 | MODEL 1 |
| MPOS_BARCODE_QRCODE_MODEL2 | 205 | MODEL 2 |

• int alignment: Alignment value.

| MPosAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ALIGNMENT_LEFT | 0 | Align left |
| MPOS_ALIGNMENT_CENTER | 1 | Align center |
| MPOS_ALIGNMENT_RIGHT | 2 | Align right |

• int moduleSize: Module size (Range: 1 ≤ moduleSize ≤ 8)

• int eccLevel: Error correction level

| MPosQRCodeECCLevel Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_QRCODE_ECC_LEVEL_L | 48 | LEVEL L |
| MPOS_QRCODE_ECC_LEVEL_M | 49 | LEVEL M |
| MPOS_QRCODE_ECC_LEVEL_Q | 50 | LEVEL Q |
| MPOS_QRCODE_ECC_LEVEL_H | 51 | LEVEL H |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-14 printPDF417

Prints the PDF417 code.

[Syntax]

Task<int> printPDF417(string data, int symbol, int alignment, int columnNumber,

int rowNumber, int moduleWidth, int moduleHeight, int eccLevel);

[Parameters]

• string data: The PDF417 code data

• int symbol: The type of the PDF417 code

| MPos2dCodeType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_PDF417 | 201 | PDF417 Standard |
| MPOS_BARCODE_PDF417_SIMPLIFIED | 202 | PDF417 Simplified |

• int alignment: Alignment value.

| MPosAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ALIGNMENT_LEFT | 0 | Align left |
| MPOS_ALIGNMENT_CENTER | 1 | Align center |
| MPOS_ALIGNMENT_RIGHT | 2 | Align right |

• int columnNumber: Number of columns (Range: 0 ≤ columnNumber ≤ 30)

• int rowNumber: Number of rows (Range: 3 ≤ rowNumber ≤ 90)

• int moduleWidth: Module width (Range: 1 ≤ moduleWidth ≤ 4)

• int moduleHeight: Module height (Range: 2 ≤moduleHeight ≤ 8)

• int eccLevel: The error correction level

| MPosPDF417ECCLevel Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_PDF417_ECC_LEVEL_0 | 48 | Error Correction Level 0 |
| MPOS_PDF417_ECC_LEVEL_1 | 49 | Error Correction Level 1 |
| MPOS_PDF417_ECC_LEVEL_2 | 50 | Error Correction Level 2 |
| MPOS_PDF417_ECC_LEVEL_3 | 51 | Error Correction Level 3 |
| MPOS_PDF417_ECC_LEVEL_4 | 52 | Error Correction Level 4 |
| MPOS_PDF417_ECC_LEVEL_5 | 53 | Error Correction Level 5 |
| MPOS_PDF417_ECC_LEVEL_6 | 54 | Error Correction Level 6 |
| MPOS_PDF417_ECC_LEVEL_7 | 55 | Error Correction Level 7 |
| MPOS_PDF417_ECC_LEVEL_8 | 56 | Error Correction Level 8 |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-15 checkPrinterStatus

Gets the printer’s current status.

[Syntax]

Task<uint> checkPrinterStatus(void);

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

| MPosPrinterStatus Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_STATUS_NORMAL | 0 | Printing is possible |
| MPOS_STATUS_PRINTER_COVEROPEN | 1 | Cover open |
| MPOS_STATUS_PRINTER_PAPEREMPTY | 2 | No paper |
| MPOS_STATUS_PRINTER_PAPER_NEAREND | 4 | Insufficient paper |
| MPOS_STATUS_PRINTER_ERROR | 8 | Error (Offline or unknown error) |
| MPOS_STATUS_BATT_HIGH | 16 | Battery level is high |
| MPOS_STATUS_BATT_MID | 32 | Battery level is middle |
| MPOS_STATUS_BATT_LOW | 64 | Battery level is low |
| MPOS_STATUS_PRINTER_OFFLINE | 128 | Offline by the printer's power off |

6-2-16 checkBattStatus

Gets the printer’s current battery level.

|  | This method is available only for “SPP-Q300”, “SPP-Q302” or mobile printers. |
| --- | --- |

[Syntax]

Task<uint> checkBattStatus(void);

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

| MPosPrinterStatus Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_STATUS_NORMAL | 0 | Battery Power Level : Full |
| MPOS_STATUS_BATT_HIGH | 16 | Battery Power Level : High |
| MPOS_STATUS_BATT_MID | 32 | Battery Power Level : Middle |
| MPOS_STATUS_BATT_LOW | 64 | Battery Power Level : Low |

6-2-17 asbEnable

Enable or disable ASB (Automatic Status Back).

|  | If enable, whenever printer status changes, the method registered by ‘statusUpdateEvent’ delegate is called. For more information about the printer status values, refer to 5-2-2. |
| --- | --- |

[Syntax]

Task<uint> asbEnable (bool enable);

[Parameters]

• bool enable: start or stop notifying of a printer status by ‘statusUpdateEvent’ delegate.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-18 printLine

Prints the straight line.

|  | • This method is available in page mode, and print direction must be left to right (no rotation), and only straight line can be printed. • This method is available only for mobile printers. |
| --- | --- |

[Syntax]

Task<int> printLine (int x1, int y1, int x2, int y2, int thickness);

[Parameters]

• int x1: x-coordinate, in dot unit, of the start point of the straight line.

• int y1: y-coordinate, in dot unit, of the start point of the straight line.

• int x2: x-coordinate, in dot unit, of the end point of the straight line.

• int y2: y-coordinate, in dot unit, of the end point of the straight line.

• int thickness: Thickness of the straight line (Range: 0 ≤ thickness ≤ 16)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-19 printBox

Prints the rectangle.

|  | • This method is available in page mode, and print direction must be left to right (no rotation). • This method is available only for mobile printers. |
| --- | --- |

[Syntax]

Task<int> printBox (int left, int top, int right, int bottom, int thickness);

[Parameters]

• int left: x-coordinate, in dot unit, of the upper-left corner of the rectangle.

• int top: y-coordinate, in dot unit, of the upper-left corner of the rectangle.

• int right: x-coordinate, in dot unit, of the lower-right corner of the rectangle.

• int bottom: y-coordinate, in dot unit, of the lower-right corner of the rectangle.

• int thickness: Thickness of the rectangle (Range: 0 ≤ thickness ≤ 16)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-20 printPdfFile

Prints PDF file specified by a local path.

[Syntax]

Task<int> printPdfFile (object context, string filePath, int startPage, int endPage, int width, int alignment, int brightness, bool isDithering, bool isCompress);

[Parameters]

• object context: not used

|  | Set this as null. |
| --- | --- |

• string filePath: The path of the PDF file.

• int startPage: The number of start page to print.

|  | The first page start with zero (0) |
| --- | --- |

• int endPage: The number of last page to print.

|  | To print all pages, use a value of -1. |
| --- | --- |

• int width: Image width to be printed or original width ot the image.

| MPosImageWidth Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_IMAGE_WIDTH_ASIS | -2 | Print as original image size |

• int alignment: Alignment.

| MPosAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ALIGNMENT_LEFT | 0 | Align to the left |
| MPOS_ALIGNMENT_CENTER | 1 | Align at the center |
| MPOS_ALIGNMENT_RIGHT | 2 | Align to the right |

• int brightness: The brightness level

| Platform | Brightness Range |
| --- | --- |
| Android, UWP | 0 ≤ brightness ≤ 100 |
| iOS | -100 ≤ brightness ≤ 100 |

• bool isDithering: Whether or not to apply dithering

• bool isCompress: Whether or not to compress data

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-21 printPdfFile

Prints PDF file specified by a local path.

[Syntax]

Task<int> printPdfFile (object context, string filePath, int startPage, int endPage,

int width, int alignment, int threshold, int didtheringType, int compressType);

[Parameters]

• object context: not used

|  | Set this as null. |
| --- | --- |

• string filePath: The path of the PDF file.

• int startPage: The number of start page to print.

|  | The first page start with zero (0) |
| --- | --- |

• int endPage: The number of last page to print.

|  | To print all pages, use a value of -1. |
| --- | --- |

• int width: Image width to be printed or original width ot the image.

| MPosImageWidth Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_IMAGE_WIDTH_ASIS | -2 | Print as original image size |

• int alignment: Alignment.

| MPosAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ALIGNMENT_LEFT | 0 | Align to the left |
| MPOS_ALIGNMENT_CENTER | 1 | Align at the center |
| MPOS_ALIGNMENT_RIGHT | 2 | Align to the right |

• int threshold: Threshold (0 ≤ threshold ≤ 255)

• int ditheringType: dithering type (0: No dithering, 1: dithering)

• int compressType: Compress type

| MPosCompressType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_IMAGE_COMPRESS_NONE | 0 | No compress |
| MPOS_IMAGE_COMPRESS_RLE | 1 | RLE |
| MPOS_IMAGE_COMPRESS_LZMA | 2 | LZMA |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-22 cutPaper

Cuts the paper.

|  | Call this method only on printers equipped with auto-cutters. |
| --- | --- |

[Syntax]

Task<int> cutPaper (bool feed, bool fullCut);

[Parameters]

• bool feed: No paper feed or paper feed to the cutting position.

• bool fullCut: Full cut or partial cut.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

6-2-23 openDrawer

Opens the cash drawer.

|  | Call this method only on printers equipped with cash drawer. |
| --- | --- |

[Syntax]

Task<int> openDrawer (int pinNumber, int onTimePulse, int offTimePulse);

[Parameters]

• int pinNumber: Cash drawer PIN number

| Value | Description |
| --- | --- |
| 0 | Number 2 |
| 1 | Number 5 |

• onTimePulse: Ontime 0 ≤ onTimePulse ≤ 255

• offTimePulse: OffTime 0 ≤ onTimePulse ≤ 255

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

7. MPosLookup Reference

7-1 Overview

• The "MPosLookup" interface provides useful methods for searching and retrieving the information needed to connect devices (printers).

7-2 Methods

|  | Use the “current” property in “MPosLookupUtil” class to call the methods defined in this interface. eg) MPosLookupUtil.Current.refreshDevicesList() eg) MPosLookupUtil.Current.getDeviceList() |
| --- | --- |

7-2-1 refreshDevicesList

Start searching for devices available to connect with HOST.

[Syntax]

Task<int> refreshDevicesList (int interfaceType)

[Parameters]

• int interface type: communication type searching for devices.

| MPosInterfaceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_INTERFACE_WIFI | 1 | WiFi (wireless LAN) |
| MPOS_INTERFACE_ETHERNET | 2 | Ethernet (wired LAN) |
| MPOS_INTERFACE_BLUETOOTH | 4 | Bluetooth |
| MPOS_INTERFACE_BLE | 8 | BLE |
| MPOS_INTERFACE_USB | 16 | USB |

[Returns]

If the method succeeds, the return value is zero(0). Other cases, returns non-zero.

7-2-2 getDevicesList

Gets the data collctions containing the connection information searched by

“refreshDeviceList” method.

[Parameters]

• int interface type: communication type for getting device list

| MPosInterfaceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_INTERFACE_WIFI | 1 | WiFi (wireless LAN) |
| MPOS_INTERFACE_ETHERNET | 2 | Ethernet (wired LAN) |
| MPOS_INTERFACE_BLUETOOTH | 4 | Bluetooth |
| MPOS_INTERFACE_BLE | 8 | BLE |
| MPOS_INTERFACE_USB | 16 | USB |

[Syntax]

ObservableCollection<MposConnectionInformation> getDevicesList (int interfaceType)

[Returns]

ObservableCollection<MposConnectionInformation>: data collctions containing the connection information.

class MposConnectionInformation { public MPosInterfaceType IntefaceType { get; set; } public string Name { get; set; } public string Address public string MacAddress { get; set; } public string IpAddress { get; set; } public string PortNumber { get; set; } public string BluetoothDeviceId { get; set; } public string USBName { get; set; } public string ProductName { get; set; } } Properties • InterfaceType: The communication type supported. • Name: The display name for the device. • Address: The address for the device (IP Address, S/N, or MAC address) • MACAddress: MAC address or S/N for Bluetooth device. • IPAddress: IP address for TCP communication. • PortNumber: Port number for TCP communication. • BluetoothDeviceId: Bluetooth deivce ID • USBName: Device name for USB communication • ProductName: USB device name

8. MPosControllerLabelPrinter Reference

8-1 Overview

• The MPosControllerLabelPrinter interface provides methods for controlling a Label printer.

8-2 Methods

8-2-1 setTextEncoding

Sets the codepage used for encoding characters.

[Syntax]

int setTextEncoding (int textEncoding);

[Parameters]

• int textEncoding: Code page to use when encoding a character string.

| MPosLabelCodePage Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_CODEPAGE_CP437 | 437 | USA, Standard Europe (PC437) |
| MPOS_CODEPAGE_CP850 | 850 | Western European (PC850) |
| MPOS_CODEPAGE_CP852 | 852 | Latin 2 (PC852) |
| MPOS_CODEPAGE_CP860 | 860 | Portuguese (PC860) |
| MPOS_CODEPAGE_CP863 | 863 | Canadian-French (PC863) |
| MPOS_CODEPAGE_CP865 | 865 | Nordic (PC865) |
| MPOS_CODEPAGE_WPC1252 | 1252 | Latin 1 (WPC1252) |
| MPOS_CODEPAGE_CP857 | 857 | Turkce (PC857) |
| MPOS_CODEPAGE_CP737 | 737 | Greek (PC 737) |
| MPOS_CODEPAGE_WPC1250 | 1250 | Central European (PC1250) |
| MPOS_CODEPAGE_WPC1253 | 1253 | Greek (WPC1253) |
| MPOS_CODEPAGE_WPC1254 | 1254 | Turkce (WPC1254) |
| MPOS_CODEPAGE_CP855 | 855 | Cyrillic (PC855) |
| MPOS_CODEPAGE_CP862 | 862 | Hebrew DOS code (PC862) |
| MPOS_CODEPAGE_CP866 | 866 | Cyrillic #2 (PC866) |
| MPOS_CODEPAGE_WPC1251 | 1251 | Cyrillic (WPC1251) |
| MPOS_CODEPAGE_WPC1255 | 1255 | Hebrew New code (WPC1255) |
| MPOS_CODEPAGE_CP928 | 928 | Greek (PC928) |
| MPOS_CODEPAGE_CP775 | 775 | Baltic (PC775) |
| MPOS_CODEPAGE_WPC1257 | 1257 | Baltic (WPC1257) |
| MPOS_CODEPAGE_CP858 | 858 | Euro (PC858) |
| MPOS_CODEPAGE_CHINA_GB2312 | 936 | Simplified Chinese (GB2312) |
| MPOS_CODEPAGE_CHINA_BIG5 | 950 | Traditional Chinese (BIG5) |
| MPOS_CODEPAGE_KOREA_KS5601 | 949 | Korean (KS5601) |
| MPOS_CODEPAGE_JAPAN_SHIFTJIS | 932 | Japanese (SHIFT JIS) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-2 setCharacterset

Sets the codepage and international characterset used for encoding characters.

[Syntax]

Task<int> setCharacterset (int characterset, int internationalCharset);

[Parameters]

• int characterset: Code page to use when encoding a character string

| MPosLabelCodePage Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_CODEPAGE_CP437 | 437 | USA, Standard Europe (PC437) |
| MPOS_CODEPAGE_CP850 | 850 | Western European (PC850) |
| MPOS_CODEPAGE_CP852 | 852 | Latin 2 (PC852) |
| MPOS_CODEPAGE_CP860 | 860 | Portuguese (PC860) |
| MPOS_CODEPAGE_CP863 | 863 | Canadian-French (PC863) |
| MPOS_CODEPAGE_CP865 | 865 | Nordic (PC865) |
| MPOS_CODEPAGE_WPC1252 | 1252 | Latin 1 (WPC1252) |
| MPOS_CODEPAGE_CP857 | 857 | Turkce (PC857) |
| MPOS_CODEPAGE_CP737 | 737 | Greek (PC 737) |
| MPOS_CODEPAGE_WPC1250 | 1250 | Central European (PC1250) |
| MPOS_CODEPAGE_WPC1253 | 1253 | Greek (WPC1253) |
| MPOS_CODEPAGE_WPC1254 | 1254 | Turkce (WPC1254) |
| MPOS_CODEPAGE_CP855 | 855 | Cyrillic (PC855) |
| MPOS_CODEPAGE_CP862 | 862 | Hebrew DOS code (PC862) |
| MPOS_CODEPAGE_CP866 | 866 | Cyrillic #2 (PC866) |
| MPOS_CODEPAGE_WPC1251 | 1251 | Cyrillic (WPC1251) |
| MPOS_CODEPAGE_WPC1255 | 1255 | Hebrew New code (WPC1255) |
| MPOS_CODEPAGE_CP928 | 928 | Greek (PC928) |
| MPOS_CODEPAGE_CP775 | 775 | Baltic (PC775) |
| MPOS_CODEPAGE_WPC1257 | 1257 | Baltic (WPC1257) |
| MPOS_CODEPAGE_CP858 | 858 | Euro (PC858) |
| MPOS_CODEPAGE_CHINA_GB2312 | 936 | Simplified Chinese (GB2312) |
| MPOS_CODEPAGE_CHINA_BIG5 | 950 | Traditional Chinese (BIG5) |
| MPOS_CODEPAGE_KOREA_KS5601 | 949 | Korean (KS5601) |
| MPOS_CODEPAGE_JAPAN_SHIFTJIS | 932 | Japanese (SHIFT JIS) |

• int internationalCharset: International character set

| MPosICS Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ICS_USA | 0 | USA Code |
| MPOS_ICS_FRANCE | 1 | FRANCE Code |
| MPOS_ICS_GERMANY | 2 | GERMANY Code |
| MPOS_ICS_UK | 3 | UK Code |
| MPOS_ICS_DENMARK_I | 4 | DENMARK1 Code |
| MPOS_ICS_SWEDEN | 5 | SWEDEN Code |
| MPOS_ICS_ITALY | 6 | ITALY Code |
| MPOS_ICS_SPAIN_I | 7 | SPAIN1 Code |
| MPOS_ICS_JAPAN | 8 | JAPAN Code |
| MPOS_ICS_NORWAY | 9 | NORWAY Code |
| MPOS_ICS_DENMARK_II | 10 | DENMARK 2 Code |
| MPOS_ICS_SPAIN_II | 11 | SPAIN 2 Code |
| MPOS_ICS_LATIN_AMERICA | 12 | LATIN AMERICA Code |
| MPOS_ICS_KOREA | 13 | KOREA Code |
| MPOS_ICS_SLOVENIA_CROATIA | 14 | SLOVENIA Code |
| MPOS_ICS_CHINA | 15 | CHINA Code |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-3 checkPrinterStatus

Gets the printer’s current status.

[Syntax]

Task<uint> checkPrinterStatus (void);;

[Returns]

| MPosLabelPrinterStatus Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_STATUS_IDLE | 0 | Printing is possible |
| MPOS_LABEL_STATUS_PRINTER_PAPEREMPTY | 1 | No paper |
| MPOS_LABEL_STATUS_PRINTER_COVEROPEN | 2 | Cover is open |
| MPOS_LABEL_STATUS_PRINTER_CUTTER_JAMMED | 4 | Cutter error |
| MPOS_LABEL_STATUS_PRINTER_TPH_OVERHEAT | 8 | TPH overheating |
| MPOS_LABEL_STATUS_PRINTER_GAP_ERROR | 16 | GAP sensor error |
| MPOS_LABEL_STATUS_PRINTER_RIBBON_END | 32 | No ribbon |
| MPOS_LABEL_STATUS_PRINTER_ON_BUILDING _LABEL_TO_BE_PRINTED | 64 | Building label buffer |
| MPOS_LABEL_STATUS_PRINTER_ON_PRINTING _LABEL | 128 | Printing |
| MPOS_LABEL_STATUS_PRINTER_ISSUED_LABEL _ISPAUSED | 256 | Paper stuck to filler |
| MPOS_LABEL_STATUS_PRINTER_WAIT_PAPER_TAKEN | 2048 | Printing pauses |
| MPOS_LABEL_STATUS_PRINTER_RFID_WRITING_ERROR | 4096 | RFID writing error |

8-2-4 printBuffer

Starts printing the contents saved in the printer

[Syntax]

Task<int> printBuffer (int numberOfCopies);

[Parameters]

• int numberOfCopies: The number of copies (Range: 1 to 65535)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-5 printRawData

Sends user-defined data to the printer.

[Syntax]

Task<int> printRawData(byte[] data);

[Parameters]

• byte[] data: Data to be sent

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-6 drawTextDeviceFont

Saves data of the character strings to the printer buffer using the bitmap font.

[Syntax]

Task<int> drawTextDeviceFont (string data, int xPos, int yPos, char fontSelection,

int fontWidth, int fontHeight, int rightSpace, int rotation, bool reverse, bool bold, bool rightToLeft, int alignment);

[Parameters]

• string data: data for characters to be printed

• int xPos: x axis coordinates of the character string

• int yPos: y axis axis coordinates of the character string

• char fontSelection: font selection

| MPosLabelBitmapFont Enumeration | Value |  | Description |
| --- | --- | --- | --- |
| MPOS_LABEL_DEVICE_FONT_6PT | ‘0’ | 9 X 15 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_8PT | ‘1’ | 12 X 20 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_10PT | ‘2’ | 16 X 25 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_12PT | ‘3’ | 19 X 30 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_15PT | ‘4’ | 24 X 38 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_20PT | ‘5’ | 32 X 40 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_30PT | ‘6’ | 48 X 76 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_14PT | ‘7’ | 22 X 34 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_18PT | ‘8’ | 28 X 44 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_24PT | ‘9’ | 37 X 58 (dots) |  |
| MPOS_LABEL_DEVICE_FONT_KOREAN1 | ‘a’ | 16 X 16 (dots) (English. Numbers 9 X 15) |  |
| MPOS_LABEL_DEVICE_FONT_KOREAN2 | ‘b’ | 24 X 24 (dots) (English. Numbers 12 X 24) |  |
| MPOS_LABEL_DEVICE_FONT_KOREAN3 | ‘c’ | 20 X 20 (dots) (English. Numbers 12 X 20) |  |
| MPOS_LABEL_DEVICE_FONT_KOREAN4 | ‘d’ | 26 X 26 (dots) (English. Numbers 16 X 30) |  |
| MPOS_LABEL_DEVICE_FONT_KOREAN5 | ‘e’ | 20 X 26 (dots) (English. Numbers 16 X 30) |  |
| MPOS_LABEL_DEVICE_FONT_KOREAN6 | ‘f’ | 38 X 38 (dots) (English. Numbers 22 X 34) |  |
| MPOS_LABEL_DEVICE_FONT_GB2312 | ‘m’ | 24 X 24 (dots) (English. Numbers 12 X 24) |  |
| MPOS_LABEL_DEVICE_FONT_BIG5 | ‘n’ | 24 X 24 (dots) (English. Numbers 12 X 24) |  |
| MPOS_LABEL_DEVICE_FONT_SHIFT_JIS | ‘j’ | 24 X 24 (dots) (English. Numbers 12 X 24) |  |

• int fontWidth: The width expansion ratio of a character. (Range: 1 to 4)

• int fontHeight: The height expansion ratio of a character. (Range: 1 to 4)

• int rightSpace: The space on the right of a character (default value: 0)

• int rotation: The printing direction of characters

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degre (clockwise) |

• bool reverse: prints characters with reverse property or not

• bool bold: prints characters with bold property or not

• bool rightToLeft: direction of printing characters (left to right or right to left)

• int alignment: alignment

| MPosLabelAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ALIGNMENT_LEFT | 0 | Align to the left |
| MPOS_LABEL_ALIGNMENT_RIGHT | 1 | Align to the right |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-7 drawTextVectorFont

Saves data of the character strings to the printer buffer using the vector font.

[Syntax]

Task<int> drawTextVectorFont (string data, int xPos, int yPos, char fontSelection,

int fontWidth, int fontHeight, int rightSpace, int rotation, bool reverse, bool bold, bool italic, bool rightToLeft, int alignment)

[Parameters]

• string data: data for characters to be printed

• int xPos: x axis coordinates of the character string

• int yPos: y axis axis coordinates of the character string

• char fontSelection: font selection

| MPosLabelVectorFont Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_VECTOR_FONT_ASCII | ‘U’ | ASCII (1Byte code) |
| MPOS_LABEL_VECTOR_FONT_KS5601 | ‘K’ | KS5601 (2Byte code) |
| MPOS_LABEL_VECTOR_FONT_BIG5 | ‘B’ | BG5 (2Byte code) |
| MPOS_LABEL_VECTOR_FONT_GB2312 | ‘G’ | GB2312 (2Byte code) |
| MPOS_LABEL_VECTOR_FONT_SHIFT_JIS | ‘J’ | Shift-JIS (2Byte code) |
| MPOS_LABEL_VECTOR_FONT_OCR_A | ‘a’ | OCR-A (1Byte code) |
| MPOS_LABEL_VECTOR_FONT_OCR_B | ‘b’ | OCR-B (1Byte code) |

• int fontWidth: character width

• int fontHeight: character height

• int rightSpace: right space of character

• int rotation: printing direction

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degre (clockwise) |

• bool reverse: prints characters with reverse property or not

• bool bold: prints characters with bold property or not

• bool italic: prints characters with italic property or not

• bool rightToLeft: direction of printing characters (left to right or right to left)

• int alignment: alignment

| MPosLabelAlignment Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ALIGNMENT_LEFT | 0 | Align to the left |
| MPOS_LABEL_ALIGNMENT_RIGHT | 1 | Align to the right |
| MPOS_LABEL_ALIGNMENT_CENTER | 2 | Align at the center |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-8 drawBarcode1D

Saves data of the 1d barcode to the printer buffer.

[Syntax]

Task<int> drawBarcode1D(string data, int xPos, int yPos, int barcodeType, int

widthNarrow, int widthWide, int height, int hri, int quietZoneWidth, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int barcodeType: Type of barcode

• int widthNarrow: Narrow bar width.

• int widthWide: Wide bar width

• int height: Barcode height

• int hri: HRI (Human Readable Interface) printing position

| Code | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_TEXTNONE | 0 | No HRI print |
| MPOS_BARCODE_TEXTBELOW | 1 | above barcode |
| MPOS_BARCODE_TEXTABOVE | 2 | below barcode |
| MPOS_BARCODE_TEXTBELOW_FONTSIZE2 | 3 | above barcode (font size 2) |
| MPOS_BARCODE_TEXTABOVE_FONTSIZE2 | 4 | below barcode (font size 2) |
| MPOS_BARCODE_TEXTBELOW_FONTSIZE3 | 5 | above barcode (font size 3) |
| MPOS_BARCODE_TEXTABOVE_FONTSIZE3 | 6 | below barcode (font size 3) |
| MPOS_BARCODE_TEXTBELOW_FONTSIZE4 | 7 | above barcode (font size 4) |
| MPOS_BARCODE_TEXTABOVE_FONTSIZE4 | 8 | below barcode (font size 4) |

• int quietZoneWidth: Quiet zone width (Range of 0 to20)

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-9 drawBarcodeMaxiCode

Saves data of the Maxicode barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeMaxiCode (string data, int xPos, int yPos, int mode)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int mode: Maxicode mode

| MPosLabelMaxicodeMode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_MAXICODE_MODE_0 | 0 | MaxiCode Mode 0 |
| MPOS_LABEL_MAXICODE_MODE_2 | 2 | MaxiCode Mode 2 |
| MPOS_LABEL_MAXICODE_MODE_3 | 3 | MaxiCode Mode 3 |
| MPOS_LABEL_MAXICODE_MODE_4 | 4 | MaxiCode Mode 4 |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-10 drawBarcodePDF417

Saves data of the PDF417 barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodePDF417(string data, int xPos, int yPos, int maximumRowCount,

int maximumColumnCount, int errorCorrectionLevel, int dataCompressionMethod, bool hri, int barcodeOriginPoint, int moduleWidth, int barHeight, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int maximumRowCount: Number of barcode Rows. (Range: 3 to 90)

• int maximumColumnCount: Number of barcode Columns. (Range: 1 to 30)

• int errorCorrectionLevel: Error correction level (Recognition rate when a part of a barcode is damaged)

| MPosLabelPDF417ECL Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL0 | 0 | Error correction level 0 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL1 | 1 | Error correction level 1 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL2 | 2 | Error correction level 2 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL3 | 3 | Error correction level 3 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL4 | 4 | Error correction level 4 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL5 | 5 | Error correction level 5 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL6 | 6 | Error correction level 6 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL7 | 7 | Error correction level 7 |
| MPOS_LABEL_ERROR_CORRECTION_LEVEL8 | 8 | Error correction level 8 |

• int dataCompressionMethod: data compression method

| PosLabelPDF41Compress Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_PDF417_DATA _COMPRESSION_TEXT | 0 | 2 characters per codeword |
| MPOS_LABEL_PDF417_DATA _COMPRESSION_NUMERIC | 1 | 2.93 characters per codeword |
| MPOS_LABEL_PDF417_DATA _COMPRESSION_BINARY | 2 | 1.2 characters per codeword |

• int hri: HRI (Human Readable Interface) printing position

| MPosLabelHRI Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_TEXTNONE | 0 | Does not print |
| MPOS_BARCODE_TEXTBELOW | 2 | Print below barcode |

• int startPosition: starting point of a barcode (default value: 1)

| MPosLabelPDF41StartPosition Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_BARCODE_ORIGIN_POINT_CENTER | 0 | Coordinate value based on middle of barcode |
| MPOS_LABEL_BARCODE_ORIGIN_POINT_UPPER | 1 | Coordinate value based on top left of barcode |

• int moduleWidth: width of the barcode module. (Range: 2 to 9)

• int barHeight: height of the bar. (Range: 4 to 99)

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-11 drawBarcodeQRCode

Saves data of the QRCode barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeQRCode (string data, int xPos, int yPos, int size, int model,

int errorCorrectionLevel, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int size: QRCode size (Range of 1 to 9)

• int model: QRCode Model

| MPosLabelQRCodeModel Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_QRCODE_MODEL_1 | 1 | Model 1 |
| MPOS_LABEL_QRCODE_MODEL_2 | 2 | Model 2 |

• int errorCorrectionLevel: Error correction level

| MPosLabelQRCodeECL 열거형 | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_QRCODE_ECCLEVEL_L | ‘L’ | Error correction rate is configured to 7% |
| MPOS_LABEL_QRCODE_ECCLEVEL_M | ‘M’ | Error correction rate is configured to 15% |
| MPOS_LABEL_QRCODE_ECCLEVEL_Q | ‘Q’ | Error correction rate is configured to 25% |
| MPOS_LABEL_QRCODE_ECCLEVEL_H | ‘H’ | Error correction rate is configured to 35% |

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-12 drawBarcodeDataMatrix

Saves data of the DataMatrix barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeDataMatrix (string data, int xPos, int yPos, int size, bool reverse,

int rotation);

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int size: DataMatrix size (Range of 1 to 4)

• int reverse: barcode reverse or normal

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-13 drawBarcodeAztec

Saves data of the Aztec barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeAztec (string data, int xPos, int yPos, int size, bool

extended Channel, int errorCorrectionLevel, bool menuSymbol, int numberOfSymbols, string optionalID, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int size: Aztec size (Range of 1 to 10)

• bool extendedChannel: Extended channel interpretation code

• int errorCorrectionLevel: Error correction level

| Code | Description |
| --- | --- |
| 0 | Automatically configures the error correction level. |
| 1~99 | Directly enter the error correction level. |
| 101~104 | 1~4 layer compact symbol |
| 201~232 | 1~32 layer compact symbol |
| 300 | Simple Aztec ”Rune” |

• bool menuSymbol: Configures the menu symbol

• int numberOfSymbols: Number of symbols for structured append (Range of 1 to 26)

• string optionalID: Optional ID field for structured append: ID field string (Maximum 24 letters)

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-14 drawBarcodeCode49

Saves data of the Code49 barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeCode49(string data, int xPos, int yPos, int widthNarrow,

int widthWide, int height, int hri, int startingMode, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int widthNarrow: Width of Narrow Bar

• int widthWide: Width of Wide Bar

• int height: Barcode height

• int hri: HRI (Human Readable Interface) printing position.

| MPosLabelHRI Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_TEXTNONE | 0 | Does not print |
| MPOS_BARCODE_TEXTABOVE | 1 | Print above barcode |
| MPOS_BARCODE_TEXTBELOW | 2 | Print below barcode |

• int startingMode: Starting Mode of a barcode

| MPosLabelCode49Mode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_STARTINGMODE_REGULAR_ALPHANUMERIC | 0 | Regular Alphanumeric Mode |
| MPOS_LABEL_STARTINGMODE_MULTIPLE_READ_ALPHANUMERIC | 1 | Multiple Read Alphanumeric |
| MPOS_LABEL_STARTINGMODE_REGULAR_NUMERIC | 2 | Regular Numeric Mode |
| MPOS_LABEL_STARTINGMODE_GROUP_ALPHANUMERIC | 3 | Group Alphanumeric Mode |
| MPOS_LABEL_STARTINGMODE_REGULAR_ALPHANUMRIC_SHIFT1 | 4 | Regular Alphanumeric Shift 1 |
| MPOS_LABEL_STARTINGMODE_REGULAR_ALPHANUMRIC_SHIFT2 | 5 | Regular Alphanumeric Shift 2 |
| MPOS_LABEL_STARTINGMODE_AUTOMATIC | 7 | Automatic Mode |

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-15 drawBarcodeCodaBlock

Saves data of the CodaBlock barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeCodaBlock (string data, int xPos, int yPos, int widthNarrow,

int widthWide, int height, bool securityLevel, int numberOfCharactersPerRow, char mode,

int numberOfRowToEncode)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int widthNarrow: Width of Narrow Bar

• int widthWide: Width of Wide Bar

• int height: Barcode height

• bool securityLevel: Barcode security level

• int numberOfCharactersPerRow: Number of characters per row(Range of 2 to 62)

• char mode: Codablock mode

| MPosLabelCodaBlockMode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_CODABLOCK_MODE_A | ‘A’ | Creates a code that uses the Code 39 character set |
| MPOS_LABEL_CODABLOCK_MODE_E | ‘E’ | Creates a code that uses the Code 128 character set |
| MPOS_LABEL_CODABLOCK_MODE_F | ‘F’ | Creates a code that uses the Code 128 character set and that automatically has func1 added to |

• int numberOfRowToEncode: The following values can be used for each mode.

| MPosLabelCodaBlockMode Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_CODABLOCK_MODE_A | ‘A’ | Can use a number between 1 and 18 |
| MPOS_LABEL_CODABLOCK_MODE_E’ | ‘E’ | Can use a number between 2 and 4 |
| MPOS_LABEL_CODABLOCK_MODE_F | ‘F’ | Can use a number between 2 and 4 |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-16 drawBarcodeMicroPDF

Saves data of the MicroPDF barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeMicroPDF (string data, int xPos, int yPos, int moduleWidth,

int moduleHeight, int mode, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int moduleWidth: module width (Range of 2 to 8)

• int moduleHeight: barcode height (Range of 1 to 99)

• int mode: barcode mode (Range of 0 to 33)

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-17 drawBarcodeIMB

Saves data of the IMB barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeIMB (string data, int xPos, int yPos, bool hri, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• bool hri: Determines whether or not to print HRI (Human Readable Interface)

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-18 drawBarcodeMSI

Saves data of the MSI barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeMSI (string data, int xPos, int yPos, int narrowWidth, int widthWide, int height, int checkDigitSelection, bool printCheckDigit, int hri, int rotation);

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int widthNarrow: Width of Narrow Bar

• int widthWide: Width of Wide Bar

• int height: Barcode height

• int checkDigitSelection: Check digit mode

| MPosLabelMSICheckDigit Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_CHECKDIGIT_NONE | 0 | Check digit not configured |
| MPOS_LABEL_CHECKDIGIT_1MOD10 | 1 | 1 Mod 10 |
| MPOS_LABEL_CHECKDIGIT_2MOD10 | 2 | 2 Mode 10 |
| MPOS_LABEL_CHECKDIGIT_1MOD11_AND_1MOD_10 | 3 | 1 Mod 11 and Mod 10 |

• bool printCheckDigit: Determines whether or not to print check digit

• int hri: HRI (Human Readable Interface) printing position.

| MPosLabelHRI Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_TEXTNONE | 0 | Does not print |
| MPOS_BARCODE_TEXTABOVE | 1 | Print above barcode |
| MPOS_BARCODE_TEXTBELOW | 2 | Print below barcode |

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-19 drawBarcodePlessey

Saves data of the Plessey barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodePlessey (string data, int xPos, int yPos, int narrowWidth,

int widthWide, int height, bool printCheckDigit, int hri, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int widthNarrow: Width of Narrow Bar

• int widthWide: Width of Wide Bar

• int height: Barcode height

• bool printCheckDigit: Determines whether or not to print check digit

• int hri: HRI (Human Readable Interface) printing position.

| Code | Value | Description |
| --- | --- | --- |
| MPOS_BARCODE_TEXTNONE | 0 | Does not print |
| MPOS_BARCODE_TEXTABOVE | 1 | Print above barcode |
| MPOS_BARCODE_TEXTBELOW | 2 | Print below barcode |

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-20 drawBarcodeTLC39

Saves data of the TLC39 barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeTLC39(string data, int xPos, int yPos, int narrowWidth,

int widthWide, int height, int rowHeightOfMicroPDF417, int narrowWidthOfMicroPDF417, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int narrowWidth: Width of Narrow Bar

• int widthWide: Width of Wide Bar

• int height: Barcode height

• int rowHeightOfMicroPDF417: Height of Micro PDF417’s row (Range of 1 to 255)

• int narrowWidthOfMicroPDF417: Width of Micro PDF417’s Narrow bar (Range of 1 to 10)

• int rotation: Printing direction of barcode to be printed

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-21 drawBarcodeRSS

Saves data of the RSS barcode to the printer buffer.

[Syntax]

Task<int> drawBarcodeRSS (string data, int xPos, int yPos, int barcodeType,

int magnification, .int separatorHeight, int barcodeHeight, int segmentWidth, int rotation)

[Parameters]

• string data: data of the barcode

• int xPos: x axis coordinates of barcode

• int yPos: y axis coordinates of barcode

• int barcodeType: Configures the barcode type.

| MPosLabelRSSType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_RSS_BARCODE_TYPE_RSS14 | 0 | RSS14 |
| MPOS_LABEL_RSS_BARCODE_TYPE_RSS14_TRUNCATED | 1 | RSS14 truncated |
| MPOS_LABEL_RSS_BARCODE_TYPE_RSS14_STACKED | 2 | RSS14 stacked |
| MPOS_LABEL_RSS_BARCODE_TYPE_RSS14_STACKED_OMNIDIRECTIONAL | 3 | RSS14 Stacked omnidirectional |
| MPOS_LABEL_RSS_BARCODE_TYPE_RSS_LIMITED | 4 | RSS limited |
| MPOS_LABEL_RSS_BARCODE_TYPE_RSS_EXPANDED | 5 | RSS Expanded |
| MPOS_LABEL_RSS_BARCODE_TYPE_UPC_A | 6 | RSS UPC A |
| MPOS_LABEL_RSS_BARCODE_TYPE_UPC_E | 7 | RSS UPC E |
| MPOS_LABEL_RSS_BARCODE_TYPE_EAN13 | 8 | EAN13 |
| MPOS_LABEL_RSS_BARCODE_TYPE_EAN8 | 9 | EAN 8 |
| MPOS_LABEL_RSS_BARCODE_TYPE_UCC_EAN128_CC_A_B | 10 | EAN128 CC-A/B |
| MPOS_LABEL_RSS_BARCODE_TYPE_UCC_EAN128_CC_C | 11 | EAN128 CC-C |

• int magnification: Can use a number between 1 and 10

• int separatorHeight: Configures the height of an RSS barcode identifier

(Range of 0 to 22)

• int barcodeHeight: Configures the barcode height

• int segmentWidth: Configures segment width (even number between 0 and 22)

• int rotation: printing direction of barcode

| MPosLabelRotation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_ROTATION_DEGREES_0 | 0 | No rotated |
| MPOS_LABEL_ROTATION_DEGREES_90 | 1 | Rotated 90 degree (clockwise) |
| MPOS_LABEL_ROTATION_DEGREES_180 | 2 | Rotated 180 degree |
| MPOS_LABEL_ROTATION_DEGREES_270 | 3 | Rotated 270 degree (clockwise) |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-22 drawBlock

Saves data of a quadrangle or line to the printer buffer.

[Syntax]

Task<int> drawBlock (int startPosX, int startPosY, int endPosX, int endPosY, string option,

int thickness)

[Parameters]

• int startPosX: x axis coordinates of top left

• int startPosY: y axis coordinates of top left

• int endPosX: x axis coordinates of bottom right

• int endPosY: y axis coordinates of bottom right

• char option: drawing option

| MPosLabelBlock Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_DRAW_BLOCK_OPTION_LINE_OVERWRITING | ‘O’ | Repeat drawing on places where lines overlap |
| MPOS_LABEL_DRAW_BLOCK_OPTION_LINE_EXCLUSIVE_OR | ‘E’ | Do not draw on places where lines overlap |
| MPOS_LABEL_DRAW_BLOCK_OPTION_LINE_DELETE | ‘D’ | Delete line |
| MPOS_LABEL_DRAW_BLOCK_OPTION_SLOPE | ‘S’ | Diagonal line |
| MPOS_LABEL_DRAW_BLOCK_OPTION_BOX | ‘B’ | Quadrangle edge |

• int thickness: Selects line thickness. This only applies if the ‘option’ value is ‘S’ or ‘B’.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-23 drawCircle

Saves data of a circle to the printer buffer.

[Syntax]

Task<int> drawCircle (int startPosX, int startPosY, int sizeSelection, int multiplier)

[Parameters]

• int startPosX: x axis coordinates of starting point* of circle region

Add starting point (intersection point of tangent of left and top) description

• int startPosY: y axis coordinates of starting point* of circle region

* Starting point: Intersection point of tangent of left and top

Starting point: (100,200)

Circle size

• int sizeSelection: size of circle

| MPosLabelCircle Enumeraton | Value | W x H Size (dots) |
| --- | --- | --- |
| MPOS_LABEL_DRAW_CIRCLE_SIZE_40X40 | 1 | 40 40 |
| MPOS_LABEL_DRAW_CIRCLE_SIZE_56X56 | 2 | 56 56 |
| MPOS_LABEL_DRAW_CIRCLE_SIZE_72X72 | 3 | 72 72 |
| MPOS_LABEL_DRAW_CIRCLE_SIZE_88X88 | 4 | 88 88 |
| MPOS_LABEL_DRAW_CIRCLE_SIZE_104X104 | 5 | 104 104 |
| MPOS_LABEL_DRAW_CIRCLE_SIZE_168X168 | 6 | 168 168 |

• int multiplier: Expands a circle by the scaling unit (Range of 1 to 4)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-24 drawImage

Saves data of an image to the printer buffer.

[Syntax]

Task<int> drawImage (object bitmapSrc, int startPosX, int startPosY, int width,

int brightness, bool isDithering, bool isCompress)

[Parameters]

• object bitmapSrc: A platform dependent image object.

|  | iOS | UIKit.UIImage |
| --- | --- | --- |
|  | Android | Android.Graphics.Bitmap |
|  | Windows | Windows.UI.Xaml.Media.Imaging.WriteableBitmap Windows.UI.Xaml.Media.Imaging.BitmapImage (deprecated) |

• int startPosX: x axis coordinates of top left of image to be printed

• int startPosY: y axis coordinates of top left of image to be printed

• int width: Width of printed image

• int brightness: The brightness level (Range: 0 ≤ brightness ≤ 100)

| Platform | Brightness Range |
| --- | --- |
| Android, UWP | 0 ≤ brightness ≤ 100 |
| iOS | -100 ≤ brightness ≤ 100 |

• bool isDithering: Whether or not to apply dithering

• bool isCompress: Whether or not to compress data

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-25 drawImageFile

Saves data of the image file specified by the path to the printer buffer.

[Syntax]

Task<int> drawImageFile (string filePath, int startPosX, int startPosY, int width,

int brightness, bool isDithering, bool isCompress)

[Parameters]

• string filePath: The path of the image file

|  | In Android, the image file should be in the SD card. |
| --- | --- |

• int startPosX: x axis coordinates of top left of image to be printed

• int startPosY: y axis coordinates of top left of image to be printed

• int width: Width of printed image

• int brightness: The brightness level

| Platform | Brightness Range |
| --- | --- |
| Android, UWP | 0 ≤ brightness ≤ 100 |
| iOS | -100 ≤ brightness ≤ 100 |

• bool isDithering: Whether or not to apply dithering

• bool isCompress: Whether or not to compress data

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-26 setPrintingType

Sets the printing type of the printer.

|  | For Bixolon label printers, printers using the direct Thermal method cannot be configured to the Transfer Thermal method. |
| --- | --- |

[Syntax]

Task<int> setPrintingType (char printingType)

[Parameters]

• char printingType: Configures the printing typ

| MPosLabelPrintingType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_PRINTING_TYPE_DIRECT_THERMAL | ‘d’ | Direct Thermal |
| MPOS_LABEL_PRINTING_TYPE_THERMAL_TRANSFER | ‘t’ | Transfer Thermal |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-27 setMargin

Sets the margins in the printing area.

[Syntax]

Task<int> setMargin (int horizontalMargin, int verticalMargin)

[Parameters]

• int horizontalMargin: Left margin

• int verticalMargin: Top margin

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-28 setLength

Sets the label length, gap, media type, and offset.

[Syntax]

Task<int> setLength (int labelLength, int gapLength, char mediaType, int offsetLength)

[Parameters]

• int labelLength: Media length

• int gapLength: Gap length of the media

• char mediaType: Media type

| MPosLabelMediaType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_MEDIA_TYPE_GAP | ‘G’ | Gap paper |
| MPOS_LABEL_MEDIA_TYPE_CONTINUOUS | ‘C’ | Continuous paper |
| MPOS_LABEL_MEDIA_TYPE_BLACK_MARK | ‘B’ | Black mark paper |

• int offsetLength: Offset length

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-29 setWidth

Sets the print width.

[Syntax]

Task<int> setWidth (int labelWidth)

[Parameters]

• int labelWidth: print width

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-30 setSpeed

Sets the printing speed.

[Syntax]

Task<int> setSpeed (int speed)

[Parameters]

• int speed: printing speed

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-31 setDensity

Sets the print density.

[Syntax]

Task<int> setDensity (int density)

[Parameters]

• int density: print density (Range of 1 to 20)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-32 setOrientation

Sets the printing orientation of the label.

[Syntax]

Task<int> setOrientation (char orientation)

[Parameters]

• char orientation: Printing orientation

| MPosLabelOrientation Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_PRINTING_DIRECTION _TOP_TO_BOTTOM | ‘T’ | Print from top to bottom |
| MPOS_LABEL_PRINTING_DIRECTION _BOTTOM_TO_TOP | ‘B’ | Print from bottom to top |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-33 setOffset

Sets offset length between black mark (or gap) and dotted lines.

[Syntax]

Task<int> setOffset (int offset)

[Parameters]

• int offset: offset value (Range of -100 to 100)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-34 setCuttingPosition

Sets the cutting position or tear-off position after printing labels.

[Syntax]

Task<int> setCuttingPosition (int cuttingPosition)

[Parameters]

• int position: Cutting position value (Range of -100 to 100).

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-35 setAutoCutter

Sets whether to use auto cutter.

|  | This method only works if an auto cutter is equipped in the printer. |
| --- | --- |

[Syntax]

long setAutoCutter (bool enable, int period)

[Parameters]

• bool enable: Determines whether or not to use auto cutter

• int period: Configures the cutting cycle. When configured to 2, it will cut every second paper.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-36 getModelName

Gets the printer’s model name.

[Syntax]

Task<int> getModelName (Action<string> modelDelegate)

[Parameters]

• Action<string> modelDelegate: A delegate with a parameter used for getting a model name.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-37 getFirmwareVersion

Gets the printer’s firmware version.

[Syntax]

Task<int> getFirmwareVersion (Action<string> versionDelegate)

[Parameters]

• Action<string> versionDelegate: A delegate with a parameter used for getting a firmware version.

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-38 getPrinterInformation

Gets the printer’s mechanical information

[Syntax]

Task<int> getPrinterInformation (int item, Action<string> infoDelegate)

[Parameters]

• int item: Mechanical information to request to the printer

| MPosLabelMechanicalInformation Enumeration | Value | Description | Unit |
| --- | --- | --- | --- |
| MPOS_LABEL_TPH_TEMPERATURE | 0 | TPH temperature | °C |
| MPOS_LABEL_PRINTING_DENSITY | 1 | Print density | - |
| MPOS_LABEL_TEAR_OFF_CUTTER_POSITION | 2 | Tear off/Cutter position | dot |
| MPOS_LABEL_MOTOR_TEMPERATURE | 3 | Motor temperature | °C |
| MPOS_LABEL_BOARD_TEMPERATURE | 4 | Board temperature | °C |
| MPOS_LABEL_BATTERY_VOLTAG | 5 | Battery voltage(%) | V(%) |

• Action<string> infoDelegate: A delegate with a parameter used for getting a mechanical information

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-39 setupRFID

Set the RFID transponder type, the number of retry codings, the number of retry labels,

and the signal strength.

[Syntax]

Task<int> setupRFID (int rfidType, int numberOfRetries, int numberofLabel, int

radioPower)

[Parameters]

• int rfidType: RFID transponder Type

| MPosLabelRfidTransponderType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_RFID_TRANSPONDER_TYPE_NONE | 0 | none |
| MPOS_LABEL_RFID_TRANSPONDER_TYPE_18000_6_TYPE_A | 1 | ISO 18000-6 Type A |
| MPOS_LABEL_RFID_TRANSPONDER_TYPE_18000_6_TYPE_B | 2 | ISO 18000-6 Type B |
| MPOS_LABEL_RFID_TRANSPONDER_TYPE_CLASS_0 | 3 | EPC Class 0 |
| MPOS_LABEL_RFID_TRANSPONDER_TYPE_CLASS_1 | 4 | EPC Class 1 |
| MPOS_LABEL_RFID_TRANSPONDER_TYPE_CLASS_1_GEN2 | 5 | EPC Class 1 Generation 2 |

• int numberOfRetries: Number of retries when coding fails (Range: 1 ~ 10)

• int numberOfLabel: Number of retry labels when coding fails (Range: 1 ~ 5)

• int radioPower: Signal strength (Range: 0 ~ 30)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-40 setRFIDPosition

Set the coding position directly on the RFID label.

[Syntax]

Task<int> setRFIDPosition (int transPosition)

[Parameters]

• int transPosition: Coding position of RFID label (y-axis value)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-41 setRFIDPassword

Set RFID Access Password and Kill Password.

[Syntax]

Task<int> setRFIDPassword (string oldAccessPwd, string oldKillPwd, string

newAccessPwd, string newKillPwd)

[Parameters]

• string oldAccessPwd: Access Password currently in use

• string oldKillPwd: Kill Password currently in use

• string newAccessPwd: Access Password to be modified

• string newKillPwd: Kill Password to modify

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-42 setEPCDataStructure

Define the EPC data structure for writing EPC data.

[Syntax]

Task<int> setEPCDataStructure (int totalSize, string fieldSizes)

[Parameters]

• int totalSize: Total bits in the field

• string fieldSizes: Bits of each field (Comma (',') is used for the separator)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-43 writeRFID

Write RFID label.

[Syntax]

Task<int> writeRFID(int dataType, int startingBlockNumber, int dataLength, string data)

[Parameters]

• int numberOfRetries: data Type

| MPosLabelRfidDataType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_LABEL_RFID_DATA_TYPE_ASCII | ‘A’ | ASCII |
| MPOS_LABEL_RFID_DATA_TYPE_HEX | ‘H’ | HEX |
| MPOS_LABEL_RFID_DATA_TYPE_EPC | ‘E’ | EPC |
| MPOS_LABEL_RFID_DATA_TYPE_USER_HEX | ‘U’ | User area hex |

|  | must be used after setEPCDataStructure. |
| --- | --- |

• int startingBlockNumber: Block to start writing (Range: 4 ~ 10)

• int dataLength: Number of bytes to write

• int data: Data (Input according to dataType)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-44 lockRFID

Lock Kill / Access / EPC Data using Access Password.

|  | Be sure to use it after running setRFIDPassword. |
| --- | --- |

[Syntax]

Task<int> lockRFID()

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

8-2-45 drawPDFFile

Saves data of the PDF file specified by the path to the printer buffer.

[Syntax]

Task<int> drawPDFFile (string filename, int startPosX, int startPosY, int width, int pageNumber, int threshold, int ditheringType, int compressType)

[Parameters]

• string filePath: The path of the PDF file.

• int startPosX: x axis coordinates of top left of image to be printed

• int startPosY: y axis coordinates of top left of image to be printed

• int width: Image width to be printed or original width ot the image.

| MPosImageWidth Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_IMAGE_WIDTH_ASIS | -2 | Print as original image size |

• int pageNumber: Page number to print.

|  | The first page start with zero (0) |
| --- | --- |

• int threshold: Threshold (0 ≤ threshold ≤ 255)

• int ditheringType: dithering type (0: No dithering, 1: dithering)

• int compressType: Compress type

| MPosCompressType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_IMAGE_COMPRESS_NONE | 0 | No compress |
| MPOS_IMAGE_COMPRESS_RLE | 1 | Compress: RLE |
| MPOS_IMAGE_COMPRESS_LZMA | 2 | Compress: LZMA |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

9. MPosControllerBCD Reference

9-1 Overview

• The MPosControllerBCD interface provides methods for controlling display devices

connected to the printer.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

9-2 Method

9-2-1 displayString

Displays the string on the display device.

[Syntax]

int displayString(string data, int codePage, int internationalCharacterSet, int line);

[Parameters]

• string data: Character string to be printed

• int codePage: Code page to use when encoding a character string.

| MPosLabelCodePage Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_CODEPAGE_CP437 | 437 | USA, Standard Europe (PC437) |
| MPOS_CODEPAGE_CP850 | 850 | Western European (PC850) |
| MPOS_CODEPAGE_CP852 | 852 | Latin 2 (PC852) |
| MPOS_CODEPAGE_CP860 | 860 | Portuguese (PC860) |
| MPOS_CODEPAGE_CP863 | 863 | Canadian-French (PC863) |
| MPOS_CODEPAGE_CP865 | 865 | Nordic (PC865) |
| MPOS_CODEPAGE_WPC1252 | 1252 | Latin 1 (WPC1252) |
| MPOS_CODEPAGE_CP857 | 857 | Turkce (PC857) |
| MPOS_CODEPAGE_CP737 | 737 | Greek (PC 737) |
| MPOS_CODEPAGE_WPC1250 | 1250 | Central European (PC1250) |
| MPOS_CODEPAGE_WPC1253 | 1253 | Greek (WPC1253) |
| MPOS_CODEPAGE_WPC1254 | 1254 | Turkce (WPC1254) |
| MPOS_CODEPAGE_CP855 | 855 | Cyrillic (PC855) |
| MPOS_CODEPAGE_CP862 | 862 | Hebrew DOS code (PC862) |
| MPOS_CODEPAGE_CP866 | 866 | Cyrillic #2 (PC866) |
| MPOS_CODEPAGE_WPC1251 | 1251 | Cyrillic (WPC1251) |
| MPOS_CODEPAGE_WPC1255 | 1255 | Hebrew New code (WPC1255) |
| MPOS_CODEPAGE_CP928 | 928 | Greek (PC928) |
| MPOS_CODEPAGE_CP775 | 775 | Baltic (PC775) |
| MPOS_CODEPAGE_WPC1257 | 1257 | Baltic (WPC1257) |
| MPOS_CODEPAGE_CP858 | 858 | Euro (PC858) |
| MPOS_CODEPAGE_CHINA_GB2312 | 936 | Simplified Chinese (GB2312) |
| MPOS_CODEPAGE_CHINA_BIG5 | 950 | Traditional Chinese (BIG5) |
| MPOS_CODEPAGE_KOREA_KS5601 | 949 | Korean (KS5601) |
| MPOS_CODEPAGE_JAPAN_SHIFTJIS | 932 | Japanese (SHIFT JIS) |

• int internationalCharset: International character set

| MPosICS Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_ICS_USA | 0 | USA Code |
| MPOS_ICS_FRANCE | 1 | FRANCE Code |
| MPOS_ICS_GERMANY | 2 | GERMANY Code |
| MPOS_ICS_UK | 3 | UK Code |
| MPOS_ICS_DENMARK_I | 4 | DENMARK1 Code |
| MPOS_ICS_SWEDEN | 5 | SWEDEN Code |
| MPOS_ICS_ITALY | 6 | ITALY Code |
| MPOS_ICS_SPAIN_I | 7 | SPAIN1 Code |
| MPOS_ICS_JAPAN | 8 | JAPAN Code |
| MPOS_ICS_NORWAY | 9 | NORWAY Code |
| MPOS_ICS_DENMARK_II | 10 | DENMARK 2 Code |
| MPOS_ICS_SPAIN_II | 11 | SPAIN 2 Code |
| MPOS_ICS_LATIN_AMERICA | 12 | LATIN AMERICA Code |
| MPOS_ICS_KOREA | 13 | KOREA Code |
| MPOS_ICS_SLOVENIA_CROATIA | 14 | SLOVENIA Code |
| MPOS_ICS_CHINA | 15 | CHINA Code |

• int line: line number to display

1 ≤ line ≤ 4 (BCD-2000)

1 ≤ line ≤ 2 (BCD-3000)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

9-2-2 cleanScreen

Deletes everything on the display device.

[Syntax]

int cleanScreen();

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

9-2-3 storeImage

Downloads images to the display device.

[Syntax]

int storeImage(ImageData data, int width, int imageNumbe);

[Parameters]

• ImageData data: A platform dependent image object.

|  | iOS | UIKit.UIImage |
| --- | --- | --- |
|  | Android | Android.Graphics.Bitmap |
|  | Windows | Windows.UI.Xaml.Media.Imaging.WriteableBitmap Windows.UI.Xaml.Media.Imaging.BitmapImage (deprecated) |

• int width: image width

-1 or 1 ≤ width ≤ 240 (BCD-2000)

-1 or 1 ≤ width ≤ 160 (BCD-3000)

• int imageNumber: index for saving images

1 ≤ imageNumber ≤ 5

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

9-2-4 storeImageFile

Downloads images to the display device.

[Syntax]

int storeImageFile(string fileName, int width, int imageNumbe);

[Parameters]

• string path: The path of the image file.

|  | In Android, the image file should be in the SD card. |
| --- | --- |

• int width: image width

-1 or 1 ≤ width ≤ 240 (BCD-2000)

-1 or 1 ≤ width ≤ 160 (BCD-3000)

• int imageNumber: index for saving images

1 ≤ imageNumber ≤ 5

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

9-2-5 displayImage

Displays images on the display device.

[Syntax]

int displayImage (int imageNumber, int xPos, int yPos);

[Parameters]

• int imageNumber: index of images to be displayed

1 ≤ imageNumber ≤ 5

• int xPos: display X-coordinates

0 ≤ xPos < 240 (BCD-2000)

0 ≤ xPos < 160 (BCD-3000)

• int yPos: display Y-coordinates

0 ≤ yPos < 64 (BCD-2000)

0 ≤ yPos < 32 (BCD-3000)

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

9-2-6 clearImage

Deletes the images downloaded to the display device.

[Syntax]

int clearImage (int imageNumbe);

[Parameters]

• int imageNumber: index of images to be displayed

1 ≤ imageNumber ≤ 5

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

9-2-7 showCursor

Shows or hides the cursor on the display device.

[Syntax]

int showCursor (int cursor);

[Parameters]

• int cursor: show/hide cursor

| MPosCursorType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_CURSOR_DISABLE | 0 | Hides the cursor |
| MPOS_CURSOR_ENABLE | 1 | Shows the cursor |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

10. MPosControllerDallasKey Reference

10-1 Overview

• MPosControllerDallasKey interface is a class for controlling the Dallas Key connected to

the printer. MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

11. MPosControllerHID Reference

11-1 Overview

• MPosControllerHID interface is a class for controlling the HID connected to the printer.

MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

12. MPosControllerMSR Reference

12-1 Overview

• MPosControllerMSR interface is a class for controlling the MSR device connected to the

printer. MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

13. MPosControllerNFC Reference

13-1 Overview

• MPosControllerNFC interface is a class for controlling the NFC connected to the

printer. MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

14. MPosControllerRFID Reference

14-1 Overview

• MPosControllerRFID interface is a class for controlling the RFID connected to the

printer. MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

15. MPosControllerScale Reference

15-1 Overview

• MPosControllerScale interface is a class for controlling the scale connected to the

printer. MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

16. MPosControllerScanner Reference

16-1 Overview

• MPosControllerScanner interface is a class for controlling the barcode scanner

connected to the printer. MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

17. MPosControllerUSBSerial Reference

17-1 Overview

• MPosControllerUSBSerial interface is a class for controlling the USB to Serial device

connected to the printer. MPosControllerDevices DataEvent can be inherited and used.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

18. MPosControllerConfig Reference

18-1 Overview

• MPosControllerConfig interface provides a method of controlling peripherals connected to the printer.

|  | Only works on printers that support B-gate or USB peripheral control commands. |
| --- | --- |

18-2 Methods

18-2-1 searchDevices

Searches for USB-type devices connected to the printer and returns the ID of the device found.

[Syntax]

int[] searchDevices();

[Returns]

Returns the ID of the device found.

18-2-2 getBGateSerialNumber

Returns the serial number of the B-gate printer.

[Syntax]

string getBGateSerialNumber();

[Returns]

Returns the B-gate's serial number.

18-2-3 getUSBDevice

Returns the PID/VID of USB devices connected to the printer.

[Syntax]

string getUSBDevice(int deviceId);

[Returns]

• int deviceId: USB device ID

[Returns]

Returns the PID and VID of the connected USB device

18-2-4 getCustomDevices

Returns information about user-defined USB devices connected to the printer.

[Syntax]

string getCustomDevices(int deviceType);

[Parameters]

• int deviceType: USB Device Type

| MPosDeviceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_DEVICE_LABEL_PRINTER | 1 | POS Printer |
| MPOS_DEVICE_PRINTER | 10 | Label Pritner |
| MPOS_DEVICE_MSR | 30 | MSR |
| MPOS_DEVICE_SCANNER | 40 | Barcord Scanner |
| MPOS_DEVICE_RFID | 60 | RFID |
| MPOS_DEVICE_DALLASKEY | 70 | Dallas Key Reader |
| MPOS_DEVICE_NFC | 80 | NFC |
| MPOS_DEVICE_DISPLAY | 110 | Customer Display |
| MPOS_DEVICE_USB_TO_SERIAL | 120 | USB to Serial |
| MPOS_DEVICE_SCALE | 130 | Scale |

[Returns]

Returns the VID/PID of all supported USB devices.

18-2-5 getSerialConfiguration

Returns the device's serial number.

[Syntax]

Int[] getSerialConfiguration(int deviceId);

[Parameters]

• int deviceId: USB device ID

| device ID | Description |
| --- | --- |
| 1 ≤ ID ≤ 9 | POS Printer |
| 11 ≤ ID ≤ 19 | Label Pritner |
| 20 ≤ ID ≤ 29 | Unregistered |
| 30 ≤ ID ≤ 39 | MSR |
| 40 ≤ ID ≤ 49 | Barcord Scanner |
| 60 ≤ ID ≤ 69 | RFID |
| 70 ≤ ID ≤ 79 | Dallas Key Reader |
| 80 ≤ ID ≤ 89 | NFC |
| 100 ≤ ID ≤ 109 | Unregistered USB UART Device |
| 110 ≤ ID ≤ 119 | Customer Display |
| 120 ≤ ID ≤ 129 | USB to Serial |
| 130 ≤ ID ≤ 139 | Scale |

[Returns]

Returns the device's baud rate, data bits, stop bits, parity bits, and flow control.

18-2-6 setSerialConfiguration

Change the serial information for that device.

[Syntax]

int setSerialConfiguration(int deviceId, int baudRate, int dataBit, int stopBit, int parityBit, int flowControl);

[Parameters]

• int deviceId: USB Device ID

• int baudRate: Baud Rate

| MPosBaudRateType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_BAUDRATE_2400 | 0 | 2400 bps |
| MPOS_BAUDRATE_4800 | 1 | 4800 bps |
| MPOS_BAUDRATE_9600 | 2 | 9600 bps |
| MPOS_BAUDRATE_19200 | 3 | 19200 bps |
| MPOS_BAUDRATE_38400 | 4 | 38400 bps |
| MPOS_BAUDRATE_57600 | 5 | 57600 bps |
| MPOS_BAUDRATE_115200 | 6 | 115200 bps |
| MPOS_BAUDRATE_230400 | 7 | 230400 bps |

• int dataBit: Data bit

| MPosDataBitType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_DATABIT_7 | 0 | 7 bits |
| MPOS_DATABIT_8 | 1 | 8 bits |

• int stopBit: Stop bit

| MPosStopBitType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_STOPBIT_1 | 0 | 1 bits |
| MPOS_STOPBIT_2 | 1 | 2 bits |

• int parityBit: Parity bit

| MPosParityBitType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_PARITYBIT_NONE | 0 | None |
| MPOS_PARITYBIT_EVEN | 1 | Even |
| MPOS_PARITYBIT_ODD | 2 | Odd |

• int flowControl: Flow control

| MPosFlowControlType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_FLOWCONTROL_NONE | 0 | None |
| MPOS_FLOWCONTROL_HW | 1 | H/W |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

18-2-7 addCustomDevice

Adds USB devices.

If a USB device is not recognized as a predefined type, make sure to define it as such before using it.

[Syntax]

int addCustomDevice (int deviceType, string vid, string pid);

[Parameters]

• int deviceType: Device Type

| MPosDeviceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_DEVICE_LABEL_PRINTER | 1 | POS Printer |
| MPOS_DEVICE_PRINTER | 10 | Label Pritner |
| MPOS_DEVICE_MSR | 30 | MSR |
| MPOS_DEVICE_SCANNER | 40 | Barcord Scanner |
| MPOS_DEVICE_RFID | 60 | RFID |
| MPOS_DEVICE_DALLASKEY | 70 | Dallas Key Reader |
| MPOS_DEVICE_NFC | 80 | NFC |
| MPOS_DEVICE_DISPLAY | 110 | Customer Display |
| MPOS_DEVICE_USB_TO_SERIAL | 120 | USB to Serial |
| MPOS_DEVICE_SCALE | 130 | Scales |

• string vid: String to USB Vender ID

• string pid: String to USB Product ID

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

18-2-8 deleteCustomDevice

Deletes custom devices on the printer that can communicate with USB peripherals.

[Syntax]

int deleteCustomDevice(int deviceType, string vid, string pid);

[Parameters]

• int deviceType: Device Type

| MPosDeviceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_DEVICE_LABEL_PRINTER | 1 | POS Printer |
| MPOS_DEVICE_PRINTER | 10 | Label Pritner |
| MPOS_DEVICE_MSR | 30 | MSR |
| MPOS_DEVICE_SCANNER | 40 | Barcord Scanner |
| MPOS_DEVICE_RFID | 60 | RFID |
| MPOS_DEVICE_DALLASKEY | 70 | Dallas Key Reader |
| MPOS_DEVICE_NFC | 80 | NFC |
| MPOS_DEVICE_DISPLAY | 110 | Customer Display |
| MPOS_DEVICE_USB_TO_SERIAL | 120 | USB to Serial |
| MPOS_DEVICE_SCALE | 130 | Scales |

• string vid: String to USB Vender ID

• string pid: String to USB Product ID

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

18-2-9 reInitCustomDeviceType

Initializes custom devices saved on the printer that can communicate with USB

peripherals.

[Syntax]

int reInitCustomDeviceType (int deviceType);

[Parameters]

• int deviceType: Device Type

| MPosDeviceType Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_DEVICE_LABEL_PRINTER | 1 | POS Printer |
| MPOS_DEVICE_PRINTER | 10 | Label Pritner |
| MPOS_DEVICE_MSR | 30 | MSR |
| MPOS_DEVICE_SCANNER | 40 | Barcord Scanner |
| MPOS_DEVICE_RFID | 60 | RFID |
| MPOS_DEVICE_DALLASKEY | 70 | Dallas Key Reader |
| MPOS_DEVICE_NFC | 80 | NFC |
| MPOS_DEVICE_DISPLAY | 110 | Customer Display |
| MPOS_DEVICE_USB_TO_SERIAL | 120 | USB to Serial |
| MPOS_DEVICE_SCALE | 130 | Scales |

[Returns]

If the method succeeds, the return value is zero (0). Other cases, returns non-zero.

18-2-10 getSerialNumber

Gets the serial number of USB devices connected to the printer.

[Syntax]

String getSerialNumber (int deviceID);

[Parameters]

• int deviceID: Device ID

[Returns]

The serial number of the device

19. Appendix

19-1 Error Code Table

• The error code table below summarizes the values returned when calling the API provided in this manual.

| MPosResult Enumeration | Value | Description |
| --- | --- | --- |
| MPOS_SUCCESS | 0 | Suceess of the method Operation |
| MPOS_ALREADY_OPEN | 1 | Already Open |
| MPOS_FAIL | 1000 | Failure of the method Operation |
| MPOS_FAIL_INVALID_INTERFACE | 1001 | Not Supported Communication Interface Type |
| MPOS_FAIL_NO_OPEN | 1002 | Tried to access the device which is not opened |
| MPOS_FAIL_NOT_SUPPORT | 1003 | Not supported API by the device |
| MPOS_FAIL_INVALID_PARAMETER | 1004 | Invalid parameter for the method |
| MPOS_FAIL_NO_RESPONSE | 1005 | No response from the device |
| MPOS_FAIL_NOT_CONNECT | 1006 | Failed to connect the device |
| MPOS_FAIL_NO_FILE | 1008 | No file in the the path specified |
| MPOS_FAIL_NOT_SUPPORT_CODEPAGE | 1012 | Not supported code page |
| MPOS_FAIL_NOT_SUPPORT_ICS | 1013 | Not supported code international code page |
| MPOS_PAGEMODE_ALREADY_IN | 1014 | Already entered in page mode. |
| MPOS_TRANSACTION_ALREADY_IN | 1015 | Already entered in transactoin mode. |
| MPOS_FAIL_NOT_SUPPORT_ESCSEQ | 1017 | Not supported escape sequence |
| MPOS_FAIL_NO_DEVICE_FOUND | 5000 | No device found |
| MPOS_FAIL_NOT_PERMISSION_ALLOWD | 5001 | Failed to get permission required |
| MPOS_FAIL_NOT_SUPPORTED_IMAGE _TYPE | 5002 | Not supported file extention |
| MPOS_FAIL_IMAGE_DECODE | 5003 | Failed to decode image in BASE64 |

Revision history

| Rev. | Date | Description |
| --- | --- | --- |
| 1.00 | 06-09-2018 | New |
| 1.01 | 30-11-2018 | ‘printPdfFile’ and ‘cutPaper’ added |
|  | 04-03-2019 | APIs for label printers are added. |
| 1.02 | 18-06-2019 | - Information of the supported iOS version chaged. - Information of the supported models updated - Type of option in ‘drawBlock" API modified. |
|  | 01-08-2019 | Brightness range for iOS changed. |
|  | 26-09-2019 | Descriptions for ‘printPdfFile’ changed. |
| 1.03 | 12-03-2020 | - Information of the supported Windows version chaged. - Information of the supported models updated |
| 1.04 | 17-09-2020 | - Information of the supported models updated - APIs for label printers rfid are added |
| 1.05 | 28-10-2020 | Information of the supported models updated |
| 1.06 | 02-03-2021 | Information of the supported models updated |
| 1.07 | 20-08-2021 | - Information of the supported models updated - Descriptions for the network entitlement added |
| 1.08 | 06-10-2021 | Information of the supported modes updated |
| 1.09 | 23-03-2022 | - Information of the supported devices and platform version update. - Methods releated to printing PDF files added and updated. |
| 1.10 | 25-04-2022 | Information of the supported models updated. |
| 1.11 | 22-06-2022 | Information of the supported models updated. |
| 1.12 | 05-07-2022 | Remove setPDFLicenseKey method. |
| 1.13 | 16-08-2022 | Information of the supported models updated. |
| 1.14 | 09-01-2023 | ‘getPrinterInformation’ and ‘openDrawer’ added |
| 1.15 | 28-03-2023 | Add Android support USB API |
|  | 18-04-2023 | Add Android, iOS support BLE API |
| 1.16 | 30-08-2023 | Fix Android Minimum SDK Version Add BK3, BK5 series models |
| 1.17 | 05-01-2024 | Added USB Peripheral Class Added API in MPosControllerConfig |
| 1.18 | 31-01-2024 | Added XQ-840II, XQ-843II |